TENDER DOCUMENTATION

PUBLIC TENDER ACCORDING TO THE

OPEN PROCEDURE

No.

JN-B0410

Purchase of SD/HD studio cameras,

HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods

CONTENTS

1	INVITATION TO SUBMIT BIDS FOR THE PUBLIC TENDER	4

2	INSTRUCTIONS FOR MAKING BIDS	5

2.1	Contracting Authority of the public Tender	5

2.2	Legal ground	5

2.3	Basic rules of operation	5

3	SUBJECT OF THE PUBLIC PROCUREMENT	6

3.1	GENERAL DESCRIPTION OF EQUIPMENT	6

3.2	TECHNICAL REQUIREMENTS	6

3.2.1	GENERAL TECHNICAL REQUIREMENTS	6

3.2.2	TECHNICAL SPECIFICATIONS FOR SINGLE LOTS	7

4	BID DOCUMENTATION	34

4.1	General conditions for making bid documentation	34

4.1.1	Clarification to the tender documentation	34

4.1.2	Language	34

4.1.3	Marking	34

4.1.4	Contents of a bid	34

4.1.5	Alternative bids and variant bids	34

4.1.6	Validity of the bids	35

4.2	Documents in the bids	35

4.2.1	Data on Bidder and Bid	35

4.2.2	Contents of the bid documentation	35

4.2.3	Form »Declaration on acceptance of the public tender conditions«	35

4.2.4	Form of an OFFER  that has to contain:	35

4.2.5	Pro forma Invoice	35

4.2.6	Bidder's Statement regarding penal acts accord. to Art. 42 of ZJN-2B – for the Company	36

4.2.7	Bidder's Statement  regarding penal acts accord. to Art. 42 of ZJN-2B – for the authoriz. person	36

4.2.8	Form “Bidder’s statements, given under penal and material responsibility”	36

4.2.9	Bank Certificate of solvency	36

4.2.10	Warranty conditions of the Bidder	36

4.2.11	Declaration of the Bidder regarding the spare parts	36

4.2.12	Declaration of the Bidder - all the offered equipment is factory-new	36

4.2.13	Declaration of the Bidder regarding the software upgrades	36

4.2.14	Statement of the manufacturer or principal	36

4.2.15	Sample of the Contract	36

4.2.16	Guarantee for the seriousness of the bid	37

4.2.17	Declaration, stating the Bidder shall provide a Performance guarantee	37

4.2.18	Technical documentation, requested in item 3.2	37

5	CONTRACT AWARD CRITERIA	37

5.1	CONTRACT AWARD CRITERIA  FOR LOT A	37

5.1.1	The lowest price  - main offer	37

5.1.2	The lowest price  - optional offer 1	37

5.2	CONTRACT AWARD CRITERIA  FOR LOT B	38

5.2.1	The lowest price  - main offer	38

5.2.2	The lowest price  - optional offer 2	38

5.2.3	The lowest price  - optional offer 3	38

5.2.4	The lowest price  - optional offer 4	38

5.2.5	The lowest price  - optional offer 5	38

5.3	CONTRACT AWARD CRITERIA  FOR LOT  C	39

5.4	CONTRACT AWARD CRITERIA  FOR LOT D	39

5.4.1	The lowest price  - main offer	39

5.4.2	The lowest price  - optional offer 6	39

5.5	CONTRACT AWARD CRITERIA  FOR LOT  E	39

5.6	CONTRACT AWARD CRITERIA  FOR LOT  F	40

5.7	CONTRACT AWARD CRITERIA  FOR LOT G	40

5.7.1	The lowest price  - main offer	40

5.7.2	The lowest price  - optional offer 7	40

5.8	Two bids with equal BEST eVALUATION RESULT	40

6	REVIEW CLAIM	41

6.1	Legal base and terms for submission	41

6.2	Method of submitting the review claim	41

7	FORMS	41

DATA ON BIDDER AND BID	42

Declaration of Bidder’s acceptance of Public Procurement conditions	43

Form OFFER – LOT A – page 1	44

Form OFFER – LOT A – page 2	45

Form OFFER – LOT B – page 1	46

Form OFFER – LOT B – page 2	47

Form OFFER – LOT B – page 3	48

Form OFFER – LOT B – page 4	49

Form OFFER – LOT B – page 5	50

Form OFFER – LOT C	51

Form OFFER – LOT D– page 1	52

Form OFFER – LOT D – page 2	53

Form OFFER – LOT E	54

Form OFFER – LOT F	55

Form OFFER – LOT G– page 1	56

Form OFFER – LOT G – page 2	57

Proforma Invoice FOR LOT ……	58

Bidder’s Statement regarding penal acts accord. to Art. 42 of ZJN-2B	59

Bidder’s Statement regarding penal acts accord. to Art. 42 of ZJN-2B	60

Bidder's Statements given under penal and material responsibility	61

Declaration of the Bidder regarding the spare parts	62

Declaration of the Bidder – all the offered equipment is factory new	63

Declaration of the BIdder regarding the software upgrades	64

STATEMENT OF THE MANUFACTURER	65

STATEMENT OF THE PRINCIPAL	66

Sample Contract	67

Guarantee for the seriousness of Bid No......	78

STATEMENT regarding the Performance Guarantee	79

Explanation of the Tender documentation No. JN-B0410	80

# INVITATION TO SUBMIT BIDS FOR THE PUBLIC TENDER

Radiotelevizija Slovenija, Javni zavod,  is in compliance with 1. passage (item 1) of Article 24 of the Public Procurement  Act (ZJN-2B),  inviting the Bidders to submit their bids to tender in accordance with the open procedure for:

Purchase of SD/HD studio cameras, HD camcorders,

lenses for studio cameras and camcorders, studio and ENG tripods

Bids in three copies (one original and two copies) must be sent in sealed envelopes to the address:

“Javni zavod RTV SLOVENIJA, KOLODVORSKA 2, VLOŽIŠČE, 1550 LJUBLJANA, SLOVENIA”, with the postscript: "DO NOT OPEN - FOR  TENDER - BID FOR TENDER NO. JN-B0410 – " SD/HD studio cameras, HD camcorders, Lenses, tripod systems “. The full address of the Bidder is to be written on the envelope.

The term for acceptance of bids is until 08.00 a.m. on 02 November, 2010. Bids received after that time limit will not be counted and will be returned unopened to the Bidders.

Public opening of bids will be on 02 November, 2010 at 09.00 a.m.  on the premises of Commercial Department of Javni Zavod RTV  Slovenija, at Komenskega Street 7 in Ljubljana.

Before the public opening of the bids the present participants of the Bidders have to hand over written authorizations for participation on the public opening.

Ljubljana,  20 September, 2010                                                      Commercial Department of

RTV Slovenija

# INSTRUCTIONS FOR MAKING BIDS

## Contracting Authority of the public Tender

Radiotelevizija Slovenija, Javni zavod, Kolodvorska 2, SI - 1550 Ljubljana

VAT No.: SI29865174; Corporate registry No.: 5056497

Bids will be appraised by the General Manager appointed professional commission. The Decision on the award of a Contract will be issued by the General Manager of Javni Zavod RTV Slovenija. In the evaluation of the tender the commission will take account of the criteria that are the constituent part of the tender documentation.

The selected Bidder must enter into a signed Contract within 8 days after receiving the “Decision on the Award of a Contract”. In the case of a Bidder not entering into a signed Contract, the Contracting Authority will validate the guarantee on the seriousness of the bid.

The Contracting Authority reserves the right of variating from the published values and quantities when selecting bids, subject to available financial resources and actual needs.

In case of receiving incomplete bids, the Contracting Authority shall annul the tender. The Contracting Authority reserves the right not to choose any of the bids (according to Art. 80 of ZJN-2B).

## Legal ground

The Public Procurement of Javni Zavod RTV Slovenija (hereinafter referred to as: Contracting Authority) is carried out in accordance with the valid Public Procurement Act (ZJN-2B) and by-laws applicable and in accordance with the legislation being valid on the field of public finances and on the field that is the subject of this public tender.

## Basic rules of operation

Every Bidder’s attempt to affect the treatment of the Contracting Authority or decision on the selection shall result in rejection of his Tender.  The same applies to all attempts related to the work and decision of the Evaluation Committee.

In the period from the publication until the conclusion of the Contract the Bidder shall be not allowed to commence and perform any actions, which might prior determine the selection of a defined bid. In the period from the selection of a Bid until the Contract comes into force the Bidder shall be not allowed to commence any actions, which might cause the Contract to become void and unfulfilled. In case the procedure is stayed the Bidder shall not commence or perform any procedures, which might aggravate the invalidation or objectivity of the Review Commission.

# SUBJECT OF THE PUBLIC PROCUREMENT

## GENERAL DESCRIPTION OF EQUIPMENT

The subject of public procurement (tender) is the purchase of studio cameras, HD camcorders with additional equipment, lenses for studio cameras and camcorders, studio and ENG tripods. The equipment will be used for production of television programs on RTV Slovenia.

The offered equipment has to comply with all technical demands which are specified in Specification and technical requirements.

The tender is split up into the following Lots:

- Lot A Studio cameras

  - 1: SD studio camera heads for S-3 (4 pieces)

  - 2: HD studio camera chains for STUDIO KOPER (3 pieces)

  - Option 1: Maintenance training for studio cameras for S-KP

- Lot B: XDCAM Camcorders

  - 1: HD camcorders type 1 (11 camcorders)

  - Option 2 : Maintenance training for camcorders type 1 and type 2

  - Option 3 : Maintenance training for recorders

  - 2: HD camcorders type 2 (15 camcorders)

  - 3: HD camcorders type 3 (2 camcorders)

  - Option 4 : Maintenance training for camcorders type 3 and type 4

  - 4: HD camcorder type 4 (1 set)

  - Option 5: Additional camcorder type 5 (2 camcorders)

- Lot C: Lenses for studio cameras for S-KP under A2 (3 pieces)

- Lot D: Lenses for camcorders under lot B – wider angle of view (9 pieces)

  - 1: Lenses for camcorders under type 1 and type 4 (9 pieces)

  - 2: Lenses for camcorders under type 3 ( 2 pieces)

  - Option 6: Maintenance training for lenses of Lot D

- Lot E: Studio tripods for cameras S-KP (3 sets)

- Lot F: ENG tripods (29 sets)

- Lot G: Lenses for camcorders under lot B (21 pieces)

  - Lenses for camcorders under type 1 and type 2 (19 pieces)

  - Option 7: Maintenance training for lenses of Lot G

## TECHNICAL REQUIREMENTS

### GENERAL TECHNICAL REQUIREMENTS

#### Technical documentation

A bidder must attach in the bid all necessary technical documentation which proves the fulfilment of all requested technical requirements.

#### Standards

The offered equipment has to operate according to the PAL standards, SD (576i at 50 Hz) and HD (720p/1080i at 50 Hz):

-  ITU-R BT.470-6

-  ITU-R.BT472

-  ITU-R BT.601/65, audio embedding SMPTE 272M

-  SMPTE 292M-1998, audio embedding SMPTE 299M

-  AES-3-1992

-  AES-11-1997

All equipment powered by 230V AC must satisfy  acts and regulations valid today in the republic of Slovenia to do with  electrical powering (230V, 50 Hz)  and safety against electrical shock in accordance with SIST EN 50160,  SIST EN 60950.

Offered equipment must conform to EMC standards:

-  SIST EN 50081-1:1995

-  SIST EN 50082-2:1997

-  SIST EN 55022-2000

-  SIST EN 55024:2000

#### Rejection of a bid

The Contracting Authority shall reject the bid from further proceeding, in case the bid documentation does not fulfil all the conditions and requests, stated in this tender documentation.

### TECHNICAL SPECIFICATIONS FOR SINGLE LOTS

#### LOT A: STUDIO CAMERAS WITH ACCESSORIES

##### SD CAMERA HEADS FOR S-3

The matters of item 3.2.2.1.1 are 4 studio camera heads with 16:9 shooting capability, which will be used in the existing camera chain configuration consisting of: triax adapter CA-590P, viewfinder BVF-55E,  Camera control unit CCU-590P.

- The bid must comprises parts which are specified in the following table:

| Item | Description | Sample model or equivalent | Qty |
| --- | --- | --- | --- |
| 1 | SD Kamerna glava s 3x2/3’’ CCD senzorji | SONYBVP-E30WSPH | 4 |

- Technical requirements for camera heads

- Camera head must includes three 2/3”  CCD sensors with at least 980x582 pixels

- Camera must have 14 bit A/D conversion of the signal from sensor

- The ability of 16:9  picture format

- Horizontal resolution at least 700 TV lines in 16.9 mode

- Sensitivity must be at least F11 / 2000lux/ 90% reflectivity of white surface

- Dynamic range must be at least 600% or more

- Signal to noise ratio must be at least -63 dB or better

- Vertical smear -140 DB or better

- At least two independent adjustments of skin tone contour must be enabled

- It must be possible to use it with large lens adapter

- It must be possible to control it with master control unit

- It must have optical star filter

- Must be compatible with Triax adapter CA-570P, BVF-55CE and Camera control unit CCU-550DP.

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid.

Technical parameters of Studio camera for S-3

| CAMERA TYPE: TRIAX ADAPTER TYPE: CA-590P CAMERA CONTROL UNIT TYPE: CCU-590P REMOTE CONTROL PANEL: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | Sensor type |  | " |
| 2 | number of pixels |  | pixels |
| 3 | number of bits of A/D conversion |  | bit |
| 4 | number of bits in digital signal processing |  | bit |
| 5 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 6 | dynamic range |  | % |
| 7 | depth of modulation |  | % |
| 8 | vertical smear |  | dB |
| 9 | signal to noise ratio |  | dB |
| 10 | number of independent skin tone contour adjustments |  |  |
| 11 | number of return channels |  |  |

##### STUDIO CAMERA CHAINS FOR STUDIO KOPER

The matters of item 3.2.2.1.2 are 3 HD/SD studio camera chains, which will be used for TV studio production in regional center Koper.

- Specification  of HD/SD studio camera chains for KP

| Item | Description | Sample model or equivalent | Qty |
| --- | --- | --- | --- |
| 1 | HD/SD studio camera with 3x2/3" CCD elements | SONY HXC100 | 3 |
| 2 | stativ adapter | SONY VCT-14 | 3 |
| 3 | colour HD LCD viewfinder 6,3'' | SONY HDVFC730W | 3 |
| 4 | Black and white CRT HD viewfinder 2'' | SONY HDVF20A | 1 |
| 5 | CCU camera control unit for item 1 | SONY HXCU-100 | 3 |
| 6 | remote control panel with colour LCD touch screen for item 1 | SONY RCP-750 | 3 |
| 7 | double ear headset for intercom with XLR connector (5- pin) and good ambient noise attenuation | BEYERDYNAMIC DT 109 | 6 |
| 8 | single ear headset for intercom with XLR connector (5-pin) and good ambient noise attenuation | BEYERDYNAMIC DT 108 | 3 |
| 9 | Operation manual |  | 3 |
| 10 | Module extender set for camera and CCU |  | 1 |
| 11 | Maintenance manual |  | 1 |
| 10 | Triax cable 8.5 mm, black | Belden | 150m |
| 11 | Triax connectors for triax cable 8,5mm with elastomer sleeve, male | Fischer | 10 |
| 12 | Triax connectors for triax cable 8,5mm with elastomer sleeve, female | Fischer | 10 |
| 13 | OPTION 1: Training course for maintenance of the cameras on location of the regional centre Koper in English language with granting of the camera manufacturer certificate to the course participants All expenses of the course realization and lecturer (accommodation, daily allowances and travelling) must be included in the price. |  | 1 |

- Technical requirements:

- Studio camera that allows shooting also from a shoulder

- Camera part has to include 3x2/3’’ progressive scan CCD elements with 1920x1080 effective picture elements  for converting optical picture to TV video signal

- Camera must have 14 bit A/D converter for CCD signal

- Camera must be equipped by robust colour HD LCD viewfinder of size at least 6''

- Focus assist function for accurate focus adjustment

- Sensitivity F11 / 2000lux / 89,9% reflectivity

- S/N ratio must be at least -55 dB at 1080i HD resolution and at least -63dB at SD resolution 50Hz

- Dynamic range must be at least 600%

- Camera must allow shooting in HD (1080/50i, 1080/59.94i, 720/50P and 720/59.94P) and SD (576/50i and 480/59.94i) resolution.

- Camera must be capable to produce picture in 16:9 and 4:3 format for SD resolution and in 16:9 format for HD resolution.

- Camera must have the following video outputs: test video BNC output, digital SDI BNC output and prompter BNC output 1Vpp, 75Ohm

- Camera must include a digital triax interface

- MIC/LINE XLR input (3-pin) for channel 1 and 2

- Intercom XLR (5-pin) connector

- Monitoring audio out (stereo earphone)   and additional assistant monitoring (tracker)

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid

Technical parameters for studio camera for Koper

| CAMERA TYPE: TRIAX ADAPTER TYPE: CAMERA CONTROL UNIT TYPE: REMOTE CONTROL PANEL: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | Sensor type |  | " |
| 2 | number of pixels |  | pixels |
| 3 | number of bits of A/D conversion |  | bit |
| 4 | number of bits in digital signal processing |  | bit |
| 5 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 6 | dynamic range |  | % |
| 7 | depth of modulation |  | % |
| 8 | vertical smear |  | dB |
| 9 | signal to noise ratio |  | dB |
| 10 | number of independent skin tone contour adjustments |  |  |
| 11 | number of return channels |  |  |

Camera control unit:

- It must include the following I/O:

  - 2 HD-SDI outputs for video signal or more

  - 2 SD-SDI outputs for video signal or more

  - 2 audio outputs

  - at least two inputs for return signal (SD-SDI signal)

  - input for teleprompter signal

  - connector for 4W (four wire) intercom system

- Synchronisation to PAL reference signal must be enabled

- Input for ON-AIR signalisation for camera head and control panel must be enabled

- Transfer of the video signal over triax cable in component mode must be enabled

- It must include Fischer type of triax connector

Remote control unit

- It must have joystick for iris adjustment

- It must allow adjustment of basic camera parameters and at least the following are required: iris, master black level, RGB gains and black levels, contour level, KNEE point and slope level, saturation of colour, storing and recalling of camera parameters, adjustment of skin contours

- It must allow joystick override function ( for preview)

- It must have ON-AIR signalisation

#### LOT B: XDCAM HD CAMCORDERS

##### XDCAM CAMCORDERS OF TYPE 1 WITH ACCESSORIES

The matter of the item 3.2.2.2.1 is 12 pieces of XDCAM camcorders with additional equipment. They will be used for TV news oriented field production in Ljubljana and in regional centre Maribor.

- Specification of HD XDCAM camcorders type 1 with accessories:

| Item | Description | Sample type or equivalent | Pieces total (LJ+MB) |
| --- | --- | --- | --- |
| 1 | XDCAM HD camcorder with 3x2/3" CCD elements | SONY PMW-500 | 11(10+1) |
| 2 | HD B&W CRT 2'' viewfinder | HDVF-20A | 12(12+1) |
| 3 | Operating manual for camcorder |  | 11(10+1) |
| 4 | Stativ adapter | SONY VCT-14 | 11(10+1) |
| 5 | Software upg. For SD Rec/Ply | SONY CBK-MD01 | 11(10+1) |
| 6 | Condens. Shotgun directional microphone with windshield | SONY ECM-678 | 11(10+1) |
| 7 | On-camera light LED Litepanel | Sola ENG | 11(10+1) |
| 8 | SxS-1 PRO memory card, 16GB | SONY SBP-16 | 22(20+2) |
| 9 | Soft case Portabrace with hard protection for VF | PB CC-HD1 | 11(10+1) |
| 10 | Memory Adaptor SD card | SONY MEAD-SD01 | 11(10+1) |
| 11 | Sony Sxs Memory Card Usb Reader/writer | SONY SBAC-US10 | 6(5+1) |
| 12 | Camcordeer protection-Shoulder case Portabrace with rain cover | PB suitable model | 11(10+1) |
| 13 | Operating instructions for on-camera light |  | 11(10+1) |
| 14 | Service manual for camcorder |  | 2(1+1) |
| 15 | Battery charger and AC power adapter for camcorders For 4 Li ion batteries from item13 on the same table | Globalmediapro SCA4S | 11(10+1) |
| 16 | Batteries Li ion 160Wh, 14,4V, V-Lock mount, D-Tap connector | Globalmediapro LI160S | 33(30+3) |
| 17 | Professional XDCAM HD422 Recorder | SONY PDW-1600 | 1 |
| 18 | XDCAM external USB drive for reading and writing XDCAM files | SONY PDW-U1 | 2 |
| 19 | HD SxS PRO compact memory recorder | PMW-EX30 | 3 |
| 20 | OPTION 2: Training course for maintenance of the camcorders type 1 and type 2 on location of the camcorder manufacturer in English language with granting of the camcorder manufacturer certificate to the course participant, for 1 person All expenses of the course realization, participant’s travelling and accommodation must be included in the price. |  | 2 |
| 21 | OPTION 3: Training course for maintenance of the recorders on location of the camera manufacturer in English language with granting of the recorder manufacturer certificate to the course participant, for 1 person All expenses of the course realization, participant’s travelling and accommodation must be included in the price. |  | 2 |

- Technical requirements:

- Shoulder type of camcorder

- Camcorder must have three 2/3’’ interline CCD sensors

- Camcorder must have 14 bit A/D conversion of the signal from sensor

- Camcorder must have at least 2”  robust CRT VF and at least 3.5” colour LCD display

- Camcorder must have build in neutral density optical and colour electrical filters

- The slot for wireless microphone with D-Sub 15-pin connector must be included in camcorder body

- Over-crank and undercrank recording must be possible (1-50 frames per second) in progressive mode

- Sensitivity at least F12 / 2000lux /89.9% reflectivity of white surface (camcorder in mode 1080/50i)

- Signal to noise ratio must be at least -54 dB without noise reduction

- Signal to noise ratio must be at least -59 dB with noise reduction

- Dynamic range must be at least 600%

- Camcorder must record on memory cards, at least 2 card slots in camcorder body

- Memory card compatibility with ExpressCard/34 slot

- Camcorder must be capable to record in several HD modes (codecs required: MPEG-2 Long GOP 422-50Mbps, 420-35Mbps) with resolution support 1080/50i, 1080/25p in 720/50p)

- SD mode recording (DV codec) is required

- Camcorder must allow recording of 4 analog audio channels at sampling frequency of 48KHz, 16-bit quantisation). The high quality HD codec must support 24 bit audio

- Interval recording must be supported

- Camcorder must have build in CACHE recording feature (at least 15 seconds of audio and video signals are buffered into the camcorder’s memory and recorded on the media when pressed record)

- HD picture and audio signal and in sync proxy (A/V signal with bit rate max. 2 Mbps) must be recorded  on card media in parallel

- Camcorder must have the following outputs: 1 x HD/SD-SDI output ( BNC conector) with possibility of recorded material playback

- XLR connector for Mic/line input  (3-pin with DC voltage +48V) for first and second channel. Auto and manual level control of audio level must be possible.

- Analog audio output (channel 1,2)

- TC input/output

- Directional microphone for IT

- Monitoring output for headphones

- IEEE 1394-port which support HDV/ DV  IN and OUT

- On-camera LED light of power to 30W (equivalent to Tungsten 250W), 10-20V DC,  must include the fixture with integrated power cord (D-Tap), ball mount, 2-way barndoor, dimming from 100% to 0, focus control , carrying case, user instructions.

- Batteries for camcorders must be Li ion, no memory effect, 14,4V, 160Wh capacity, weight of 1100g, V-lock mount adapter and D-Tap connector for light powering.

- Battery charger for charging the batteries from  previous item  must have input voltage 90-240V, 47-63Hz, charging time of  160WH battery 370 minutes, can be used as a camcorder AC power adapter with output 15V, 4.5A . Battery charger must have 4 charging places.

- Batteries must have over temperature, over current and over voltage protection.

- Batteries must have LED indication of remaining capacity.

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid

Technical parameters of camcorder type 1

| CAMCORDER TYPE: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | CCD type |  |  |
| 2 | number of pixels |  | pixels |
| 3 | horizontal/vertical resolution |  | TV lines |
| 4 | A/D conversion |  | bit |
| 5 | digital signal processing |  | bit |
| 6 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 7 | minimum illumination |  | lux |
| 8 | maximum gain |  | dB |
| 9 | dynamic range |  | % |
| 10 | depth of modulation |  | % |
| 11 | vertical smear |  | dB |
| 12 | signal to noise ratio |  | dB |
| 13 | weight of body+VF+microphone |  | kg |
| 14 | power consumption of camcorder (body+VF+microphone) |  | W |

Technical requirements for professional XDCAM HD422 recorder under item 17 from the table Specification of XDCAM HD camcorders type 1 with accessories

- playback of HD materials, recorded on the exchangeable media in HD camcorder type 3 and type 4, fulfilled with the requirements from this tender documentation

- recording of HD materials on the same exchangeable media, used in HD camcorder type 3 and type 4 from this tender documentation

- recording and playback of different standard material, such as MPEG HD422, MPEG HD (HQ, SP and LP), MPEG IMX and DVCAM

- playback of materials  24 f/sek in 1080 mode

- JOG functionality and nonlinear access to recordings

- HD-SDI and SD-SDI input/output

- IEEE1394 connector for DV OUT and File Access Mode

- Gigabit Ethernet connector for file exchange

- Conversion of HD material into SD at playback and inversely

- LCD display for picture, VU-meter, TC, etc.

- Input for reference signal

- TC in/out connector

- RS 422 control connector

- Analog audio in/out (2 channels, selected from 4 possible)

- AES/EBU in/out (4 channels)

- Composite video output and audio monitoring output (2 channels)

- Stereo output for headphones

XDCAM ext. USB drive for reading and writing XDCAM files, from table index, item 18 “SPECIFICATION OF HD CAMCORDER TYPE 1 WITH ADDITIONAL EQUIPMENT”:

- Reproduction of HD/SD materials, recorded on exchangeable media in HD camcorder type 3 and type 4, fulfilled with the requirements from this tender documentation

- NLE access and transfer via USB 2 connector

- Data transfer from media to NLE without data loss

MEMORY CARD RECORDER/PLAYER, from table index, item 19 “SPECIFICATION OF HD CAMCORDER TYPE 1 WITH ADDITIONAL EQUIPMENT”:

- playback of HD materials, recorded on the exchangeable media in HD camcorder type 1 (only some of codecs) and type 2, fulfilled with the requirements from this tender documentation

- recording of HD materials on the same exchangeable media, used in HD camcorder type 1 and type 2 from this tender documentation

- recording and playback of different standard material with MPEG-2 MP@HL and MPEG-2 MP@H14 codec

- JOG functionality and nonlinear access to recordings

- HD-SDI and SD-SDI input/output (embedded audio and TC)

- IEEE1394 connector for HDV in/out

- USB-2 connector for copying from SxS cards

- HDMI output

- Conversion of HD material into SD at playback and

- LCD display for picture, VU-meter, TC, etc.

- Analog audio in/out (2 channels, selected from 4 possible)

- Composite video output and audio monitoring output (2 channels)

- Stereo output for headphones

##### XDCAM HD CAMCORDERS TYPE 2 WITH ACCESSORIES

The matter of item 3.2.2.2.2 are 15 pieces of HD/SD XDCAM camcorders with accessories. They will be  for TV news oriented field production in Ljubljana and in regional centres Maribor and Koper.

- Specification of  HD/SD camcorders type 2 with accessories:

| Item | Description | Sample type or equivalent | Pieces total (LJ+MB) |
| --- | --- | --- | --- |
| 1 | HD/SD XDCAM camcorder with 3x2/3" CMOS sensors | SONY PMW-350 | 15(12+3) |
| 2 | SD/DVCAM recording option | CBK-DV01 | 15(12+3) |
| 3 | Monochrome viewfinder 2’’ | DXF-20W | 1 |
| 4 | User manual |  | 15(12+3) |
| 5 | Stativ adapter | SONY VCT-14 | 15(12+3) |
| 6 | SxS-1 PRO memory card, 16GB | SONY SBP-16 | 30(24+6) |
| 7 | Memory Adaptor SD card | SONY MEAD-SD01 | 15(12+3) |
| 8 | Sony Sxs Memory Card Usb Reader/writer | SONY SBAC-US10 | 16(12+3+1) |
| 7 | Electret Condenser shotgun microphone with windshield | SONY ECM-678 | 15(12+3) |
| 8 | On-camera LED light Litepanel | Sola ENG | 16(13+3) |
| 9 | Soft case Portabrace with hard VF protection | PB CC-HD1 | 15(12+3) |
| 10 | Shoulder case Portabrace with rain cover | PB SC-F350 | 15(12+3) |
| 11 | On-camera light user manual |  | 16(13+3) |
| 12 | Camcorder service manual |  | 3(2+1) |
| 13 | Battery charger and AC power adapter for camcorders For Li ion batteries from item 14 on the same table | Globalmediapro SCA2S | 15(12+3) |
| 14 | Batteries Li ion 160Wh, 14,4V, V-Lock mount, D-Tap connector | Globalmediapro LI160S | 45(36+9) |

- Technical requirements:

- Shoulder type of camcorder

- Camcorder must have three 2/3’’ CMOS sensors

- Camcorder must have horizontal resolution at least 1000 lines in 1920x1080i mode

- Camcorder must have at least 3,5”   LCD viewfinder

- Built-in neutral density optical and colour electrical filters

- The slot for wireless microphone with D-Sub 15-pin connector must be included in camcorder body

- Camcorder must record on memory cards, at least 2 card slots in camcorder body

- Memory card compatibility with ExpressCard/34 slot

- Recording on memory cards: 50 minutes of  HD 35Mbps V/A on 16 GB memory card, 70 minutes of  SD  DVCAM on 16 GB memory card

- Instant recording start

- Read speed from SXS card 800Mb/s

- Recording format video MPEG-2 Long GOP

HD- HQ mode: bit rate 35Mb/s (1920x1080/50i,25p,1440x1080/50i,25p, 1280x720/50p,25p)

HD- SP mode: bit rate 25Mb/s (1440x1080/50i)

SD mode: DVCAM  (720x576/50i,25PsF)

- Recording format audio linear PCM

HD mode: 4 channels, 16-bit, 48kHZ

SD mode: 2 channels, 16-bit, 48kHz

- Camera sensitivity at least F13 / 2000lux /89.9% reflectivity of white surface (camcorder in mode 1080/50i)

- Signal to noise ratio must be at least -59 dB with noise suppression

- Camcorder must be capable to record in several HD modes (1080/50i, 1080/25p in 720/50i)

- Camcorder must allow recording of 4 channels of 16 bit audio (at sampling frequency of 48KHz)

- Interval recording must be supported

- Camcorder must have build in CACHE recording feature (15 seconds of audio and video signals are buffered into the camcorder’s memory and recorded on the media when pressed record)

- HD picture and avdio signal and metadata (camera parameters) must be recorded  on memory card in HD mode

- Camcorder must have 1 x HD/SD-SDI output ( BNC conector) with possibility of recorded material playback, 1x composite output

- XLR connector for Mic/line input  (3-pin female with DC voltage +48V) for first and second channel. Auto and manual level control of audio level must be possible.

- Analog audio output (channel 1,2)-XLR 5 pin connector

- TC input/output

- Monitoring output for headphones, stereo mini jack

- IEEE 1394-port, 4-pin which support HDV 1080i / DVCAM stream IN / OUT

- HDMI output, A-type

- USB device, Mini-B

- On-camera LED light of power to 30W (equivalent to Tungsten 250W), 10-20V DC,  must include the fixture with integrated power cord (D-Tap), ball mount, 2-way barndoor, dimming from 100% to 0, focus control , carrying case, user instructions.

- Batteries for camcorders must be Li ion, no memory effect, 14,4V, 160Wh capacity, weight of 1100g, V-lock mount adapter and D-Tap connector for light powering.

- Battery charger for charging the batteries from  previous item  must have input voltage 90-240V, 47-63Hz, charging time of  160WH battery 370 minutes, can be used as a camcorder AC power adapter with output 15V, 4.5A . Battery charger must have 4 charging places

- Batteries must have over temperature, over current and over voltage protection.

- Batteries must have LED indication of remaining capacity.

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid

Technical parameters of camcorder type 2

| CAMCORDER TYPE: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | Sensor type |  |  |
| 2 | number of pixels |  | pixels |
| 3 | horizontal/vertical resolution |  | TV lines |
| 4 | A/D conversion |  | bit |
| 5 | digital signal processing |  | bit |
| 6 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 7 | minimum illumination |  | lux |
| 8 | maximum gain |  | dB |
| 9 | dynamic range |  | % |
| 10 | depth of modulation |  | % |
| 11 | vertical smear |  | dB |
| 12 | signal to noise ratio |  | dB |
| 13 | weight of body+VF+microphone |  | kg |
| 14 | power consumption of camcorder (body+VF+microphone) |  | W |

##### XDCAM HD/SD CAMCORDERS OF TYPE 3

The matter of item 3.2.2.2.3 are 2 pieces of HD/SD XDCAM camcorders with accessories. They will be used for TV field production in regional centre Koper.

- Specification HD/SD camcorders type 3 with accessories:

| Item | Description | Sample type or equivalent | Pieces total KP |
| --- | --- | --- | --- |
| 1 | HD/SD XDCAM camcorder with 3x1/2" IT CCD sensors | SONY PDW-F335 | 2 |
| 2 | On-camera microphone with cable, mount kit |  | 2 |
| 3 | Microphone windshield |  | 2 |
| 4 | Stativ adapter | SONY VCT-14 | 2 |
| 5 | CRT viewfinder 2’’ |  | 2 |
| 6 | Soft case Portabrace for camcorder with lens | PB CC-22-PW | 2 |
| 7 | Shoulder case Portabrace with rain cover | PB SC-F350 | 2 |
| 8 | On-camera LED light Litepanel | Sola ENG | 2 |
| 9 | Battery charger (4 charging pos.)and AC power adapter for Li ion batteries from item 8 on the same table | Globalmediapro SCA4S | 2 |
| 10 | Batteries Li ion 160Wh, 14,4V, V-Lock mount, D-Tap connector | Globalmediapro LI160S | 6 |
| 11 | Camcorder user manual |  | 2 |
| 12 | Camcorder service manual |  | 1 |
| 13 | On-camera light user manual |  | 2 |
| 14 | OPTION 4: Training course for maintenance of the camcorders type 3 and type 4 on location of the camcorder manufacturer in English language with granting of the camcorder manufacturer certificate to the course participant, for one person All expenses of the course realization, participant’s travelling and accommodation must be included in the price. |  | 2 |

- Technical requirements:

- Camcorder for shooting from a shoulder

- Camera part has to include 3x1/2’’ IT CCD elements for converting the optical picture to TV video signal

- Camera part and recorder must be mounted in the same housing

- Camcorder must have 12 bit A/D converter for CCD signal

- Camcorder must be equipped by robust B&W CRT viewfinder of size at least 2''  and colour LCD display of size at least 3,5''

- Sensitivity F9 / 2000lux / 89,9% reflectivity

- S/N ratio must be at least -54 dB

- Dynamic range must be at least 400%

- Vertical smear must be at least –120dB

- Minimum brightness must be 0,004 lux or less

- Camcorder must be capable to record material on a media which can be removed and can record up to 60 min of HD 35Mb/s V/A material  from the camcorder and which can be archived for long term.

- Camcorder must allow shooting and recording in HD (1080/50i and 1080/25P) and SD (DVCAM format) resolution.

- Camcorder must be capable to produce picture in 16:9 and 4:3 format for SD resolution and in 16:9 format for HD resolution.

- Camcorder must support shooting in PAL and NTSC standard for SD resolution.

- Camcorder must allow recording of four analogue audio channels at 48 KHz sampling frequency and 16-bit digitalisation.

- Camcorder must record on media the following signals: picture in HD or SD resolution, audio, proxy of  video and audio (it must be in-sync with hi-res material) which bit-rate can be max. 2Mb/s high.

- Camcorder must allow interval recording.

- Camcorder must have build in CACHE recording feature (10 seconds of audio and video signals are buffered into the camcorder’s memory and recorded on the media when pressed record)

- Camcorder must have the following video outputs: SD analogue composite (1Vpp, 75Ohm, BNC connector) for play out of recorded DVCAM materials and HD analogue component (3 BNC connectors) for play out of recorded HD materials.

- MIC/LINE XLR input (3-pin with +48V supply voltage) for channel 1 and 2. It must be allowed manual and automatic control of recording level of audio signals.

- Analogue audio out (channel 1,2)

- TC (time code) input and output

- Monitoring audio out (stereo earphone)

- IEEE 1394-connector, which supports File Access Mode (IN and OUT) and DV OUT

- Batteries for camcorders must be Li ion, no memory effect, 14,4V, 160Wh capacity, weight of 1100g, Sony V-lock mount adapter and D-Tap connector for light powering.

- Battery charger for charging the batteries from previous item must have input voltage 90-240V, 47-63Hz, charging time of  160WH battery 370 minutes, can be used as a camcorder AC power adapter with output 15V, 4.5A . Battery charger must have 4 charging places.

- Batteries must have over temperature, over current and over voltage protection.

- Batteries must have LED indication of remaining capacity.

- On-camera LED light of power to 30W (equivalent to Tungsten 250W), 10-20V DC,  must include the fixture with integrated power cord (D-Tap), ball mount, 2-way barndoor, dimming from 100% to 0, focus control , cariing case, user instructions.

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid

Technical parameters of camcorder type 3

| CAMCORDER TYPE: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | CCD type |  |  |
| 2 | number of pixels |  | pixels |
| 3 | horizontal/vertical resolution |  | TV lines |
| 4 | A/D conversion |  | bit |
| 5 | digital signal processing |  | bit |
| 6 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 7 | minimum illumination |  | lux |
| 8 | maximum gain |  | dB |
| 9 | dynamic range |  | % |
| 10 | depth of modulation |  | % |
| 11 | vertical smear |  | dB |
| 12 | signal to noise ratio |  | dB |
| 13 | weight of body+VF+microphone |  | kg |
| 14 | power consumption of camcorder (body+VF+microphone) |  | W |

##### XDCAM HD/SD CAMCORDERS TYPE 4 WITH ACCESSORIES

The matter of the item 3.2.2.2.4 is 1 piece of XDCAM camcorders with additional equipment.  It will  be used for TV field production in Koper.

- Specification of HD XDCAM camcorder with accessories:

| Item | Description | Sample type or equivalent | Pieces total KP |
| --- | --- | --- | --- |
| 1 | XDCAM HD camcorder with 3x2/3" CCD elements | SONY PDW-F700 | 1 |
| 2 | HD B&W CRT 2'' viewfinder | HDVF-20A | 1 |
| 3 | Operating manual for camcorder |  | 1 |
| 4 | Stativ adapter | SONY VCT-14 | 1 |
| 5 | Software upg. For SD Rec/Ply | SONY CBKZ-MD01 | 1 |
| 6 | Condens. Shotgun directional microphone with windshield | SONY ECM-678 | 1 |
| 7 | On-camera light LED Litepanel | Sola ENG | 1 |
| 8 | Soft case Portabrace with hard protection for VF | PB CC-HD1 | 1 |
| 9 | Camcordeer protection-Shoulder case Portabrace with rain cover | PB SC-PDW700 | 1 |
| 10 | Operating instructions for on-camera light |  | 1 |
| 11 | Service manual for camcorder |  | 1 |
| 12 | Battery charger and AC power adapter for camcorders For 4 Li ion batteries from item13 on the same table | Globalmediapro SCA4S | 1 |
| 13 | Batteries Li ion 160Wh, 14,4V, V-Lock mount, D-Tap connector | Globalmediapro LI160S | 3 |

- Technical requirements:

- Shoulder type of camcorder

- Camcorder must have three 2/3’’ interline CCD sensors

- Camcorder must have 14 bit A/D conversion of the signal from sensor

- Camcorder must have at least 2”  robust CRT VF and at least 3.5” LCD display

- Camcorder must have build in neutral density optical and colour electrical filters

- The slot for wireless microphone with D-Sub 15-pin connector must be included in camcorder body

- Over-crank and undercrank recording must be possible (1-50 frames per second) in progressive mode

- Sensitivity at least F12 / 2000lux /89.9% reflectivity of white surface (camcorder in mode 1080/50i)

- Signal to noise ratio must be at least -54 dB without noise reduction

- Signal to noise ratio must be at least -59 dB with noise reduction

- Dynamic range must be at least 600%

- Vertical smear must be at least -135 dB or better

- Camcorder must record on optical disks

- Camcorder must be capable to record in several HD modes (1080/50i, 1080/25p in 720/50p)

- Camcorder must be capable to record in SD (MPEG IMX and DVCAM) mode

- Camcorder must allow recording of 4 channels of 16 bit audio (at sampling frequency of 48KHz), the high quality HD codec must support 24 bit audio

- Interval recording must be supported

- Camcorder must have build in CACHE recording feature (30 seconds of audio and video signals are buffered into the camcorder’s memory and recorded on the media when pressed record)

- HD picture and avdio signal and in sync proxy (A/V signal with bit rate max. 2 Mbps) must be recorded  on optical disc. In paralel proxy must be recorded on USB key.

- Optical discs must have capacity at least 50 GB

- Camcorder must have the following outputs: 2 x HD/SD-SDI output ( BNC conector) with possibility of recorded material playback

- XLR connector for Mic/line input  (3-pin with DC voltage +48V) for first and second channel. Auto and manual level control of audio level must be possible.

- Analog audio output (channel 1,2)

- TC input/output

- Very directional microphone with sensitivity -28dB (0dB=1V/Pa)

- Monitoring output for headphones

- IEEE 1394-port which support File Acess Mode (IN in OUT) and DV OUT

- 100 Mb/s Ethernet port - Tx (RJ-45)

- On-camera LED light of power to 30W (equivalent to Tungsten 250W), 10-20V DC,  must include the fixture with integrated power cord (D-Tap), ball mount, 2-way barndoor, dimming from 100% to 0, focus control , carrying case, user instructions.

- Batteries for camcorders must be Li ion, no memory effect, 14,4V, 160Wh capacity, weight of 1100g, Sony V-lock mount adapter and D-Tap connector for light powering.

- Battery charger for charging the batteries from  previous item  must have input voltage 90-240V, 47-63Hz, charging time of  160WH battery 370 minutes, can be used as a camcorder AC power adapter with output 15V, 4.5A . Battery charger must have 4 charging places

- Batteries must have over temperature, over current and over voltage protection.

- Batteries must have LED indication of remaining capacity.

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid

Technical parameters of camcorder type 4

| CAMCORDER TYPE: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | CCD type |  |  |
| 2 | number of pixels |  | pixels |
| 3 | horizontal/vertical resolution |  | TV lines |
| 4 | A/D conversion |  | bit |
| 5 | digital signal processing |  | bit |
| 6 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 7 | minimum illumination |  | lux |
| 8 | maximum gain |  | dB |
| 9 | dynamic range |  | % |
| 10 | depth of modulation |  | % |
| 11 | vertical smear |  | dB |
| 12 | signal to noise ratio |  | dB |
| 13 | weight of body+VF+microphone |  | kg |
| 14 | power consumption of camcorder (body+VF+microphone) |  | W |

##### OPTION 5: ADDITIONAL XDCAM HD/SD CAMCORDERS OF TYPE 5

| Item | Description | Sample model or equivalent | Pieces total |
| --- | --- | --- | --- |
| 1 | HD/SD XDCAM camcorder with 3x1/2’’ CMOS sensors | SONY PMW-320K | 2 |
|  | Including lens 16x5.8, servo/manual with UV protection filter |  |  |
| 2 | User manual | SONY VCT-14 | 2 |
| 3 | Stativ adapter |  | 2 |
| 4 | SxS-1 PRO memory card, 16GB | SONY SBP-16 | 1 |
| 5 | Memory Adaptor SD card | SONY MEAD-SD01 | 1 |
| 6 | On-camera light LED Litepanel 30W with user instructions | SOLA ENG | 1 |
| 7 | Battery charger and AC power adapter for camcorders For Li ion batteries from item 8 on the same table | Globalmediapro SCA2S | 1 |
| 8 | Batteries Li ion 160Wh, 14,4V, V-Lock mount, D-Tap connector | Globalmediapro LI160S | 4 |

- Technical requirements:

- Shoulder type of camcorder

- Camcorder must have three ½’’ CMOS sensors

- Camcorder must have horizontal resolution at least 1000 lines in 1920x1080i mode

- Camcorder must have at least 3,5”   LCD viewfinder

- Built-in neutral density optical and colour electrical filters

- The slot for wireless microphone with D-Sub 15-pin connector must be included in camcorder body

- Camcorder must comprise the ½’’ lens with focal lenght at least from 5,8mm to 92,8mm.

- Lens must have maqnual and auto focusing.

- Lens must have the protection lens.

- Camcorder must record on memory cards, at least 2 card slots in camcorder body

- Memory card compatibility with ExpressCard/34 slot

- Recording on memory cards: 50 minutes of  HD 35Mbps V/A on 16 GB memory card

70 minutes of  SD  DVCAM on 16 GB memory card

- Instant recording start

- Read speed from SXS card 800Mb/s

- Recording format video MPEG-2 Long GOP

HD- HQ mode: bit rate 35Mb/s (1920x1080/50i,25p,1440x1080/50i,25p, 1280x720/50p,25p)

HD- SP mode: bit rate 25Mb/s (1440x1080/50i)

SD mode: DVCAM  (720x576/50i,25PsF)

- Recording format audio linear PCM

HD mode: 4 channels, 16-bit, 48kHZ

SD mode: 2 channels, 16-bit, 48kHz

- Camera sensitivity at least F11 / 2000lux /89.9% reflectivity of white surface (camcorder in mode 1080/50i)

- Signal to noise ratio must be at least -54dB typical

- Camcorder must be capable to record in several HD modes (1080/50i, 1080/25p in 720/50i)

- Camcorder must allow recording of 4 channels of 16 bit audio (at sampling frequency of 48KHz)

- Interval recording must be supported

- Camcorder must have build in CACHE recording feature (15 seconds of audio and video signals are buffered into the camcorder’s memory and recorded on the media when pressed record)

- HD picture and avdio signal and metadata (camera parameters) must be recorded  on memory card in HD mode

- Camcorder must have 1 x HD/SD-SDI output ( BNC conector) with possibility of recorded material playback, 1x composite output, 1x analog component

- XLR connector for Mic/line input  (3-pin female with DC voltage +48V) for first and second channel. Auto and manual level control of audio level must be possible.

- Analog audio output (channel 1,2)

- TC input/output

- Monitoring output for headphones, stereo mini jack

- IEEE 1394-port, 4-pin which support File Acess Mode HDV 1080i / DVCAM stream IN / OUT

- HDMI output, A-type

- USB device, Mini-B

- On-camera LED light of power to 30W (equivalent to Tungsten 250W), 10-20V DC,  must include the fixture with integrated power cord (D-Tap), ball mount, 2-way barndoor, dimming from 100% to 0, focus control , cariing case, user instructions.

- Batteries for camcorders must be Li ion, no memory effect, 14,4V, 160Wh capacity, weight of 1100g, V-lock mount adapter and D-Tap connector for light powering.

- Battery charger for charging the batteries from  previous item  must have input voltage 90-240V, 47-63Hz, charging time of  160WH battery 370 minutes, can be used as a camcorder AC power adapter with output 15V, 4.5A . Battery charger must have 2 charging places.

- Batteries must have over temperature, over current and over voltage protection.

- Batteries must have LED indication of remaining capacity.

- The attached table TPC Form (Technical Parameters of Camera) must be filled in, signed  and returned with the bid

Technical parameters of camcorder type 5

| CAMCORDER TYPE: Manufacturer: Bidder: |  |  |  |
| --- | --- | --- | --- |
|  | Technical Parameter | Value |  |
| 1 | Sensor type |  |  |
| 2 | number of pixels |  | pixels |
| 3 | horizontal/vertical resolution |  | TV lines |
| 4 | A/D conversion |  | bit |
| 5 | digital signal processing |  | bit |
| 6 | sensitivity at 2000 lux / 90% reflectivity |  | F |
| 7 | minimum illumination |  | lux |
| 8 | maximum gain |  | dB |
| 9 | dynamic range |  | % |
| 10 | depth of modulation |  | % |
| 11 | vertical smear |  | dB |
| 12 | signal to noise ratio |  | dB |
| 13 | weight of body+VF+microphone |  | kg |
| 14 | power consumption of camcorder (body+VF+microphone) |  | W |

#### LOT C: STUDIO LENSES FOR STUDIO CAMERAS FOR KP OF ITEM 3.2.2.1.2

The offer has to include the HD EFP 2/3’’ lens of BROADCAST range. They will be used on cameras from item 3.2.2.1.2 for TV studio production in regional centre in Koper. All the lenses will be used on the cameras which will be chosen on the same tender. The mount on the lenses is expected to be bayonet mount, dependable of the chosen cameras and will be determined when ordered.

- Specification of the lens for HD/SD studio cameras for KP:

| Item | Description | Sample type or equivalent | Pieces KP |
| --- | --- | --- | --- |
|  | HD ENG/EFP OBJEKTIVI 2/3“ |  |  |
| 1 | 2/3’’ HD lenses with focal length from 7,7 mm or less to focal length 131 mm or more (without range extender). The lens must be equipped with: 2x range extender, macro, internal focusing, servo motors for zoom and focus | CANON KJ17ex7.7 IASE | 2 |
| 2 | Protection UV filter for lens from item 1 |  | 2 |
| 3 | Polarisation filter for lens from item 1 |  | 2 |
| 4 | 0,8 x wide converter with adapter for lens from item 1 | CANON W80-IIIB + CANON 85II | 2 |
| 5 | Full servo digital kit for zoom and for focus for the lens from item 1 |  | 2 |
| 6 | 2/3’’ HD lenses with focal length from 6 mm or less to focal length 78 mm or more without range extender. The lens must be equipped with: macro, internal focusing, servo motors for zoom and focus | CANON KJ13x6 KRS | 1 |
| 7 | protection (UV) filter fo lens from item 6 |  | 1 |
| 8 | Full servo digital kit for zoom and for focus for the lens from item 6 |  | 1 |

#### LOT D: LENSES FOR CAMCORDERS – wider angle of view

##### LENSES FOR CAMCORDERS TYPE 1 AND TYPE 4

The offer has to include the HD ENG/EFP 2/3’’ lens of BROADCAST range. They will be used on cameras from lot B1 for TV news oriented field production in Ljubljana and in regional centres Maribor and Koper. All the lenses will be used on the cameras from items 3.2.2.2.1 and 3.2.2.2.4 which will be chosen on the same tender. The mount on the lenses is expected to be bayonet mount, dependable of the chosen cameras and will be determined when ordered.

- Specification of the lens for SD/HD camcorders type 1 and type 4

| Item | Description | Sample type or equivalent | Pieces total (LJ+MB+KP |
| --- | --- | --- | --- |
|  | HD ENG/EFP OBJEKTIVI 2/3“ |  |  |
| 1 | 2/3’’ HD wide-angle lenses with focal length from 6,3 mm or less to focal length 100 mm or more (without range extender). The lens must be equipped with: 2x range extender, macro, internal focusing, servo motors for zoom | FUJINON HA16x6.3BERM | 9(7+1+1) |
| 2 | Protection UV filter for lens from item 1 |  | 9(7+1+1) |
| 3 | Digital Servo kit for zoom for the lens from item 1 |  | 3(1+1+1) |

##### LENSES FOR CAMCORDERS TYPE 3

The offer has to include the HD ENG/EFP ½’’ lens of BROADCAST range. They will be used on cameras from lot B2 for TV field production in the region of Koper. All the lenses will be used on the cameras from item 3.2.2.2.3 which will be chosen on the same tender. The mount on the lenses is expected to be bayonet mount, dependable of the chosen cameras and will be determined when ordered.

- Specification of the lens for SD/HD camcorders TYPE 3

| Item | Description | Sample type or equivalent | Pieces total KP |
| --- | --- | --- | --- |
|  | HD ENG/EFP OBJEKTIVI 1/2“ |  |  |
| 1 | ½’’ ENG HD lenses with focal length from 4,6mm or less to focal length 74 mm or more (without range extender). The lens must be equipped with: 2x range extender, macro, internal focusing, servo motor for zoom. | FUJINON HS16X4.6BERM | 2 |
| 2 | Protection UV filter for lens from item 1 |  | 2 |
| 3 | Polarisation filter for lens from item 1 |  | 1 |
| 4 | Digital servo kit for zoom for the lens from item 1 | FUJINON ERD-10A-D01 | 2 |

#### LOT  E:  STUDIO PEDESTALS FOR STUDIO CAMERAS FOR S-KP

The offer has to include the 2 sets of pedestals in OB version because of the need for bigger wheels. Pedestals  will be used for cameras from A2 for TV studio  production in the regional studio Koper.

- The specification  of the pedestals for cameras under A2:

| Item | Description | Sample model or equivalent | Pieces KP |
| --- | --- | --- | --- |
| 1 | Pan and tilt head for load capacity min. 43 kg, tilt range 90, flat base. It must include wedge plate and second telescopic pan bar (all together 2). | VINTEN Vector 430 | 2 |
| 2 | Two stage OB pneumatic pedestal, load capacity min. 75 kg with steerable OB dolly, min-max height 69,5-146,5 cm, 16cm wheels, brakes | VINTEN Osprey Elite OB pedestal | 2 |
| 3 | Additionall 3 OB dolly skid with 16 cm wheels for pedestal same as under item 2 |  | 1 |
| 4 | OB cable guards set for OB pedestals as under item 2 |  | 3 |
| 5 | Script holder with light and power connector for the studio camera chosen under lot A2 of the same tendernosilec |  | 7 |

- Technical requirements:

- Working height of pedestal must be at least  from 69,5 cm to 146,5 cm

- Tilt range 180°

- Pedestal must allow smooth and continuous on-shot height varying from minimum to maximum    height range

- Pedestal wheels must have a diameter of at least 16 cm

- Steering ring must allow direct control over all wheels which allow the shooting while driving

#### LOT F:  ENG TRIPOD SYSTEMS FOR CAMCORDERS TYPE 1,2,3,4 UNDER LOT B (29 sets)

The matter of the LOT E are ENG tripods which will be used for field TV production with all camcorders from LOT B of this tender. They will be used in Ljubljana, Maribor and Koper.

- The specification  and requests for the items:

| Item | Description | Sample type or equivalent | Pieces total (LJ+MB+KP) |
| --- | --- | --- | --- |
| 1 | fluid tripod head for load capacity at least 16 kg | SACHTLER System 15 SB SL MCF | 29(22+4+3) |
| 2 | single extension carbon-fibber tripod for load capacity at least 40 Kg |  | 29(22+4+3) |
| 3 | Off-ground spreader |  | 29(22+4+3) |
| 4 | 3 Rubber feet set |  | 29(22+4+3) |
| 5 | Soft carrying case |  | 29(22+4+3) |

- Technical requirements:

- Working height of the tripod at least from 67cm to 168cm.

- Transport lenght of the tripod not more than 89cm.

- Every leg of the tripod has to comprise the quick height adjustment set up clamp, which is released and locked with simple one movement of the hand (speed-lock). Clamp must be ergonomically shaped in a rounded design which ensures that cables slide off without catching.

#### LOT G: LENSES FOR CAMCORDERS

##### LENSES FOR CAMCORDERS TYPE 1 AND TYPE 2

The offer has to include the HD ENG/EFP 2/3’’ lens of BROADCAST range. They will be used on cameras from lot B2 for TV news oriented field production on different correspondent  locations in Slovenija. All the lenses will be used on the cameras from item 3.2.2.2.1 and 3.2.2.2.2 which will be chosen on the same tender. The mount on the lenses is expected to be bayonet mount, dependable of the chosen cameras and will be determined when ordered.

- Specification of the lens for SD/HD camcorders type 1 and type 2

| Item | Description | Sample type or equivalent | Pieces total (LJ+MB+KP) |
| --- | --- | --- | --- |
|  | HD ENG/EFP OBJEKTIVI 2/3“ |  |  |
| 1 | 2/3’’ ENG HD lenses with focal length from 7,6 mm or less to focal length 129 mm or more (without range extender). The lens must be equipped with: 2x range extender, macro, internal focusing, servo motors for zoom. Minimum object distance (MOD) must be 0,6m or less. | FUJINON ZA17x7.6BERM HD | 19(16+3) |
| 2 | Protection UV filter for lens from item 1 |  | 19(16+3) |
| 3 | OPTION 7: Training course for maintenance of the lens under lot G on location of RTVSLO Ljubljana in English language All expenses of the course realization and lecturer (accommodation, daily allowances and travelling) must be included in the price. |  | 1 |

#### GENERAL REQUIREMENTS FOR ALL LOTS

##### Delivery time

The Contracting Authority requires delivery date of equipment at the latest within the following time limits, counting from the beginning of the Contract validity: - 8 weeks for camcorders type 1

- 6 weeks for all other equipment

##### Quantitative acceptance

The quantitative acceptance will be done in warehouse of RTV Slovenija, Čufarjeva 6, Ljubljana. With the quantitative acceptance the Contracting Authority will determine completeness of delivered equipment regarding contract agreement.

The quantitative acceptance must be finished in four workdays after receipt of equipment in warehouse RTV Slovenija.

##### Final acceptance of equipment

It is understood that the final acceptance will be successfully done when the equipment is working according to the requested technical and functional requirements in tender documentation. At this final acceptance the representatives of the Bidder can be present.

The final acceptance of the delivered goods (equipment) must be completed by the Contracting Authority within 21 (twenty-one) days after the date of quantitative acceptance, at the latest.

All eventual incorrectness, ascertained at final acceptance procedure, are to be eliminated by the Bidder within 14 (fourteen) days, after this time limit the Contracting Authority may cash-in a performance guarantee.  It is understood that the final acceptance will be carried out when all eventual incorrectness are eliminated. The incorrectness are to be specified in writing by the Contracting Authority.

##### Warranty period

The minimal required warranty period for the whole equipment is 24 (twenty-four) months from the date of final acceptance has been carried out. The Bidders can offer longer warranty period.

##### Maintenance of equipment in warranty and after warranty period

Required is free of charge equipment maintenance in warranty period, free of charge technical support and eventual software upgrades for a warranty period, at least 24 months from the successful final acceptance.

The bidder must enclose data regarding contact details of technical support and location of technical support for user technical support and for service support.

The Bidder must enclose his own Statement with detailed information on warranty and the after warranty service support of the equipment, wherein the following must be evident: the address of the European service for bidden equipment, contact person, phone and fax no., response time and the time period when the error will be eliminated or time period when the Bidder will ensure the replacement equipment.

The Bidder must ensure in his own Statement:

-  for the warranty period and until 5 years after final acceptance: the dispatch of the replacement module by express courier service within max 2 weeks (48 hours  for spare parts which are on stock in Europe) after notice in working days. The Contract Authority will report the fault by fax or e-mail. The Contracting Authority will return the defective module after receipt of the replacement one.

##### Spare parts

The bidder has to guaranties the availability of the spare parts for the bidden equipment for the period of at least five (5) years after the final acceptance of the equipment.

# BID DOCUMENTATION

## General conditions for making bid documentation

### Clarification to the tender documentation

The Bidder can demand clarifications to tender documentation in e-mail or written  form only, using the attached form, at the latest 6 days prior to the term for acceptance of bids, to the e-mail address: javna.narocila@rtvslo.si   or  fax no. + 386 1 475 2186. Questions, not asked on the prescribed form, will not be answered.

Questions and answers shall be published on web side: www.rtvslo.si/razpisi, on sub-page JAVNA NAROČILA - PUBLIC PROCUREMENT under the mark Javno naročilo JN-B0410.

### Language

Bids have to be made in the Slovenian or English language.

### Marking

Pages of bid documentation must be numbered and bound with string so it is not possible to add pages.

Data which will be righteously marked as confidential by the Bidder, will be used only for the purposes of this public tender and will not be accessible to anyone except persons which are included in this public tender. These data will not be read on public opening of the Bids nor will they be read afterwards. Persons, included in this public tender as well as the Contracting Authority will be responsible for protection of these data.

The Bidder can mark data as confidential only if they include personal data which are not included in other public register nor otherwise acceptable to public. The Bidder can also mark as confidential business information which are marked as confidential either with regulations or internal acts of the Bidder.

The Contracting Authority will treat as confidential all data in bid documentation which will be in right upper corner marked as "CONFIDENTIAL" with large capital letters. Under this inscription there should be signature of the Bidder's official representative or authorized person. If only part of the document is confidential, the confidential part should be underlined with red colour and in the same line marked "CONFIDENTIAL" at the right edge.

The Contracting Authority will not be responsible for confidentiality of the data if these data are not marked as stated above.

The Contracting Authority will summon the Bidder to withdraw the confidential mark if the data is unjustified marked as confidential. The confidential mark can be withdrawn by the Bidder's official representative or authorized person writing "CANCELLATION" above the confidential mark. He must also write time and date of this cancellation and sign it.

In the case of a Bidder not withdrawing the confidential mark in the time, determined by Contracting Authority, the Contracting Authority will reject the complete bid.

### Contents of a bid

The Bidders can submit their bids for the equipment, required in the tender documentation:

- for all the required equipment

- for equipment according to single or several Lots, wherein all the equipment, stated in the single Lot, has to be offered.

### Alternative bids and variant bids

Alternative bids and variant bids are not allowed.

### Validity of the bids

Bids must be valid till 31 December, 2010, otherwise they will be rejected. In exceptional circumstances the Contracting Authority can demand from the Bidder to extend the validity of the bid for a certain period of time. The Contracting Authority's demand and the Bidder's replies must be in written form. The Bidder can refuse the Contracting Authority's demand. In case the Bidder complies with the demand, the Contracting Authority shall ask for the bid extension and eventually for the extension of the validity of the Warranty for the seriousness of the bid. All other conditions of the bid shall remain the same.

## Documents in the bids

The bid documentation must contain the following documents, sorted in the following order:

### Data on Bidder and Bid

### Contents of the bid documentation

### Form »Declaration on acceptance of the public tender conditions«

### Form of an OFFER  that has to contain:

Prices:

Prices must be expressed in EUR, on DDP Ljubljana basis, delivered to the Warehouse of RTV Slovenija, Čufarjeva Street 6 in Ljubljana (in accord. with INCOTERMS 2000); without VAT.

The prices must be fixed for the period of the Contract duration.

Payment conditions:

The Contracting Authority will perform the payments under the following conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 %  -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank warranty or any other instrument for insurance of the retained payments.

Optional offers:

The Bidder has to offer separately from the main offer (at the Lots where requested) also the values of the optional offers, according to the requirements in item 3.2 of the Tender documentation.

Delivery date:

The Contracting Authority requires delivery date of equipment at the latest within the following time limits, counting from the beginning of the Contract validity:

- 8 weeks for camcorders type 1

- 6 weeks for all other equipment

### Pro forma Invoice

### Bidder's Statement regarding penal acts accord. to Art. 42 of ZJN-2B – for the Company

### Bidder's Statement  regarding penal acts accord. to Art. 42 of ZJN-2B – for the authoriz. person

If the Bidder has more than one legal representatives, he must enclose “Bidder's Statement regarding penal acts accord. to Art. 42 of ZJN-2B – for the authoriz. Person” for each of them.

### Form “Bidder’s statements, given under penal and material responsibility”

### Bank Certificate of solvency

The Bidder must enclose Bank Certificate of solvency, showing the Bidder’s account has not been blocked within the last six months. On the date of opening of the bids, this Certificate must not be older than one month.

If the Bidder has several accounts, he must enclose Bank Certificate of solvency for all accounts.

The document (under item 4.2.9) may also be a photocopy. In such case the Bidder has to inscribe on each photocopied page, that he confirms the truthfulness of the stated data and sign and stamp this statement on  each photocopied page. In case the Contracting Authority subsequently requests the original evidence, the Bidder shall present the same for verification.

### Warranty conditions of the Bidder

The Bidder must enclose his own statement with the stated guarantee period and detailed information on Maintenance of equipment in warranty and after warranty period, according to the requirements in items 3.2.2.7.4 and 3.2.2.7.5 of the Tender documentation.

### Declaration of the Bidder regarding the spare parts

### Declaration of the Bidder - all the offered equipment is factory-new

### Declaration of the Bidder regarding the software upgrades

### Statement of the manufacturer or principal

In case the Bidder is not also the manufacturer of the equipment, one of the mentioned statements is to be enclosed.

If the Bidder encloses the statement of the principal, he has to enclose also the copy of the Contract between the principal and the manufacturer of the equipment, showing the principal has full access to the spare parts i.e. replacement equipment for all offered equipment.

### Sample of the Contract

Sample of the Contract, wherein each page is to be initialled and stamped by the Bidder's official representative or authorized person. Each article in the Contract is written first in Slovenian and then in English language. In case of any doubt or misunderstanding, the Slovenian version should therefore be considered final.

### Guarantee for the seriousness of the bid

The guarantee for the seriousness of the bid (issued by Bank or Insurance Company), irrevocable, unconditional, payable at first call,  valid till 31 December, 2010, in the value  EUR 28.000,00 (for all Lots), or in the following values for separate Lots:

- for Lot A : EUR   3.500,00,

- for Lot B : EUR 13.500,00,

- for Lot C : EUR   1.000,00,

- for Lot D : EUR   3.200,00,

- for Lot E : EUR   1.000,00,

- for Lot F:  EUR   2.600,00,

- for Lot G : EUR   3.200,00.

### Declaration, stating the Bidder shall provide a Performance guarantee

### Technical documentation, requested in item 3.2

The Bidder must enclose all relevant documentation, required in item 3.2, but not particularly mentioned in items from 4.2.1 to 4.2.17, as well as technical data, wherein it must be unambiguously evident that the bidden equipment fulfils all the technical requirements.

# CONTRACT AWARD CRITERIA

The criterion for  evaluation of  the bids, that will fulfil all the conditions and requirements from this tender documentation and will be technically entirely corresponding to all conditions and technical specifications, is PRICE (in Lots with optional offers the criteria is divided into sub-criteria). The Bidder, whose bid reaches maximal  number of points at each Lot, according to the below mentioned criteria, is going to be selected.

## CONTRACT AWARD CRITERIA  FOR LOT A

### The lowest price  - main offer

The price shall be calculated into the points according to the following formula (the basis shall be the total value of the lowest bid for the  “main offer” from the form Offer):

the estimated main offer

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest main offer

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – MAIN OFFER =  97

### The lowest price  - optional offer 1

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  the “optional offer ” from the form Offer):

the estimated optional offer 1

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 1

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER 1 =  3

## CONTRACT AWARD CRITERIA  FOR LOT B

### The lowest price  - main offer

The price shall be calculated into the points according to the following formula (the basis shall be the total value of the lowest bid for the  “main offer” from the form Offer):

the estimated main offer

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest main offer

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – MAIN OFFER =  88

### The lowest price  - optional offer 2

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  this optional offer from the form Offer):

the estimated optional offer 2

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 2

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER 2 =  3

### The lowest price  - optional offer 3

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  this optional offer from the form Offer):

the estimated optional offer 3

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 3

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER 3 =  3

### The lowest price  - optional offer 4

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  this optional offer from the form Offer):

the estimated optional offer 4

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 4

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER 4 =  3

### The lowest price  - optional offer 5

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  this optional offer from the form Offer):

the estimated optional offer 5

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 5

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER 5 =  3

## CONTRACT AWARD CRITERIA  FOR LOT  C

For Lot C, the Bid with the lowest bidden total value of the equipment  is going to be selected. The Bid with the lowest total value will at the evaluation receive 100 points, the remaining Bids will receive less points, expressed as percentage to the Bid with the lowest price.

the estimated offer

( 2 	-	-----------------------------------------------  )   	x   100

the lowest offer

## CONTRACT AWARD CRITERIA  FOR LOT D

### The lowest price  - main offer

The price shall be calculated into the points according to the following formula (the basis shall be the total value of the lowest bid for the  “main offer” from the form Offer):

the estimated main offer

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest main offer

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – MAIN OFFER =  97

### The lowest price  - optional offer 6

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  this optional offer from the form Offer):

the estimated optional offer 6

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 6

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER =  3

## CONTRACT AWARD CRITERIA  FOR LOT  E

For Lot E, the Bid with the lowest bidden total value of the equipment  is going to be chosen. The Bid with the lowest total value will at the evaluation receive 100 points, the remaining Bids will receive less points, expressed as percentage to the Bid with the lowest price.

the estimated offer

( 2 	-	-----------------------------------------------  )   	x   100

the lowest offer

## CONTRACT AWARD CRITERIA  FOR LOT  F

For Lot F, the Bid with the lowest bidden total value of the equipment  is going to be selected. The Bid with the lowest total value will at the evaluation receive 100 points, the remaining Bids will receive less points, expressed as percentage to the Bid with the lowest price.

the estimated offer

( 2 	-	-----------------------------------------------  )   	x   100

the lowest offer

## CONTRACT AWARD CRITERIA  FOR LOT G

### The lowest price  - main offer

The price shall be calculated into the points according to the following formula (the basis shall be the total value of the lowest bid for the  “main offer” from the form Offer):

the estimated main offer

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest main offer

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – MAIN OFFER =  97

### The lowest price  - optional offer 7

The price shall be calculated into the points according to the following formula (the basis shall be the value of the lowest  bid for  the “optional offer ” from the form Offer):

the estimated optional offer 7

( 2 	-	-----------------------------------------------  )	x	   max. points for this sub-criterion

the lowest optional offer 7

The calculation will be done on two decimal places. The bid can be appraised by 0 points at the minimum. Max. points for the sub-criterion THE LOWEST PRICE – OPTIONAL OFFER 7 =  3

## Two bids with equal BEST eVALUATION RESULT

In case two or more bids have at the end of measuring equal maximum number of points, the Contracting Authority shall choose the bid that offers longer warranty period. Should the bid can not be chosen anyway, the next measure is shorter delivery date.

# REVIEW CLAIM

## Legal base and terms for submission

Pursuant to the Act on the Review of Public Procurement Procedures (ZRPJN-UPB4, Official Gazette of the Republic of Slovenia No. 26/2007, ZRPJN-E, Official Gazette of the Republic of Slovenia No. 53/2007) the review claim may be submitted by any individual who showed or shows interest in the award of Contract and has suffered or could suffer damage due to the act of the Contracting Authority. The review claim may be submitted in all phases of public Contract award procedure and against any activity of the Contracting Authority unless otherwise provided by the law regulating the award of public Contract and the Act on the Review of the Public Procurement Procedures. After the Contract has been awarded, the term for submitting a review claim shall be ten days following the receipt of a Decision on the Award of a Contract.

The review of the Public Procurement Procedures shall be carried out according to the Act on the Review of Public Procurement Procedures, while the provisions related to the procedure of the Legal Procedure Act shall only apply unless otherwise provided by the Act on the review of the Public Procurement procedures.

## Method of submitting the review claim

The review claim must be explained. The Applicant shall forward the review claim to the Contracting Authority in two copies. Slovenian Ministry in charge of finance shall be notified by one copy of the review claim. The review claim shall be sent to the Contracting Authority through registered mail with A.R. (advice of receipt), or electronically (if the signature is verified).

The Applicant shall state the following data on the review claim:

- the Applicant of the review claim and the contact person;

- the name of the Contracting Authority;

- the public procurement (i.e. tender) and the notification on awarding the Contract or recognizing the qualifications;

- infringements;

- facts and evidences proving the infringements;

- evidence on the payment of a tax in the value of  EUR 5.000,00; made to the adequate account at the Ministry in charge of finance. The account No.: 01100-1000358802.

The Contracting Authority shall reject  all incorrect claims and claims not submitted within the above stated period.

# FORMS

The enclosed forms must be completed and attached to the Bid.

## DATA ON BIDDER AND BID

FOR PUBLIC TENDER – JN-B0410

Purchase of SD and HD studio cameras HD camcorders,

lenses for studio cameras and camcorders, studio and ENG tripods

To be completed by a Bidder. Valid data that are accessible in public and according to the regulations related to a status and legal form are obligatory. The Contracting Authority shall examine some data through electronic media available.

| BID NO.: |  |
| --- | --- |
| COMPANY NAME / NAME OF THE BIDDER: |  |
| LEGAL STATUS OF THE BIDDER: |  |
| ADDRESS OF THE BIDDER: |  |
| NUMBER OF (BUSINESS) ACCOUNT - IBAN, SWIFT: |  |
| BANK’S ADDRESS: |  |
| CONTACT PERSON RESPONSIBLE FOR A BID: |  |
| E-MAIL OF CONTACT PERSON, PHONE AND FAX: |  |
| ID VAT NO. OF A BIDDER.: |  |
| COMPANY REGISTRATION NO. OF A BIDDER.: |  |
| PERSON RESPONSIBLE FOR SIGNING A CONTRACT: |  |
| FUNCTION OF PERSON RESPONSIBLE FOR SIGNING A CONTRACT: |  |

Validity of the bid: ………………………………….(at least till 31 December, 2010)

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Declaration of Bidder’s acceptance of Public Procurement conditions

JN-B0410

The Bidder:

- Title and name of the Bidder: ………………………………………………………………

-  Address: .....................................................................................................................

-  Bidder's official representative or authorized person: …………………………………

We herewith declare, that we accept all the conditions, required in the tender documentation for the submission of our bid on the published tender for the purchase of SD and HD studio cameras HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods i.e.:

(please encircle)

- Lot A Studio cameras

- Lot B: Camcorders

- Lot C: Lenses for studio cameras

- Lot D: Lenses for camcorders – wider angle of view

- Lot E: Studio tripods

- Lot F: ENG tripods

- Lot G: Lenses for camcorders

We declare, that all data, given in our bid documentation are truthful, and that all the enclosed document photocopies equal to the originals. For given data, their truthfulness and suitability of photocopies we assume full responsibility.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

.

## Form OFFER – LOT A – page 1

FOR PUBLIC TENDER JN-B0410

STUDIO CAMERAS WITH ADITIONAL EQUIPMENT – MAIN OFFER

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT A – page 2

FOR PUBLIC TENDER JN-B0410

STUDIO CAMERAS WITH ADITIONAL EQUIPMENT – OPTIONAL  OFFER 1

| Item | Description | Value in EUR for one person |
| --- | --- | --- |
| 01 | Training course for maintenance of the cameras on location of the regional centre Koper for one (1) person -in English language with granting of the camera manufacturer certificate to the course participants (All expenses of the course realization and lecturer (accommodation, daily allowances and travelling) must be included in the price.) |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the purchase of the equipment, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

100 % within 60 days after the completed training and receipt of the relevant Invoice.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT B – page 1

FOR PUBLIC TENDER JN-B0410

XDCAM HD CAMCORDERS – MAIN OFFER

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT B – page 2

FOR PUBLIC TENDER JN-B0410

XDCAM HD CAMCORDERS – OPTIONAL  OFFER 2

| Item | Description | Value in EUR for two persons |
| --- | --- | --- |
| 01 | Training course for two persons for maintenance of the camcorders type 1 and type 2 on location of the camcorder manufacturer - in English language with granting of the camcorder manufacturer certificate to the course participants (All expenses of the course realization, participants` travelling and accommodation must be included in the price.) |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the purchase of the equipment, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

100 % within 60 days after the completed training and receipt of the relevant Invoice.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT B – page 3

FOR PUBLIC TENDER JN-B0410

XDCAM HD CAMCORDERS – OPTIONAL  OFFER 3

| Item | Description | Value in EUR for two persons |
| --- | --- | --- |
| 01 | Training course for two persons for maintenance of the recorders on location of the recorder manufacturer - in English language with granting of the recorder manufacturer certificate to the course participants, (All expenses of the course realization, participants` travelling and accommodation must be included in the price.) |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the purchase of the equipment, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

100 % within 60 days after the completed training and receipt of the relevant Invoice.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

..............................                                                                       ...........................................

## Form OFFER – LOT B – page 4

FOR PUBLIC TENDER JN-B0410

XDCAM HD CAMCORDERS – OPTIONAL  OFFER 4

| Item | Description | Value in EUR for two persons |
| --- | --- | --- |
| 01 | Training course for two persons for maintenance of the camcorders type 3 and type 4 on location of the camcorder manufacturer - in English language with granting of the camcorder manufacturer certificate to the course participants (All expenses of the course realization, participants` travelling and accommodation must be included in the price.) |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the purchase of the equipment, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

100 % within 60 days after the completed training and receipt of the relevant Invoice.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT B – page 5

FOR PUBLIC TENDER JN-B0410

XDCAM HD CAMCORDERS – OPTIONAL  OFFER 5

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of this optional offer equipment, the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT C

FOR PUBLIC TENDER JN-B0410

LENSES FOR STUDIO CAMERAS KP

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT D– page 1

FOR PUBLIC TENDER JN-B0410

LENSES FOR CAMCORDERS-WIDER ANGLE OF VIEW – MAIN OFFER

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT D – page 2

FOR PUBLIC TENDER JN-B0410

LENSES FOR CAMCORDERS – OPTIONAL  OFFER 6

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Training course for maintenance of the lenses of Lot D on the location of RTV Slovenia (All expenses of the course realization and lecturer (accommodation, daily allowances and travelling) must be included in the price.) |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the purchase of the equipment, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of  training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

100 % within 60 days after the completed training and receipt of the relevant Invoice.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT E

FOR PUBLIC TENDER JN-B0410

STUDIO TRIPODS FOR STUDIO CAMERAS KP

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT F

FOR PUBLIC TENDER JN-B0410

ENG TRIPODS FOR CAMCORDERS OF TYPE 1,2,3,4

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT G– page 1

FOR PUBLIC TENDER JN-B0410

LENSES FOR CAMCORDERS – MAIN OFFER

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Price of offered equipment, according to the requirements in item 3.2 of tender documentation |  |
| 02 | Allowed discounts |  |
| 03 | TOTAL NET - DDP Ljubljana, delivered to the warehouse of RTV Slovenija, Čufarjeva Street 6, Ljubljana (accor. to INCOTERMS 2000); without VAT |  |

Payment conditions:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 % -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

Delivery date:

Warranty:

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Form OFFER – LOT G – page 2

FOR PUBLIC TENDER JN-B0410

LENSES FOR CAMCORDERS – OPTIONAL  OFFER 7

| Item | Description | Value in EUR |
| --- | --- | --- |
| 01 | Training course for maintenance of the lenses of Lot G on the location of RTV Slovenia (All expenses of the course realization and lecturer (accommodation, daily allowances and travelling) must be included in the price.) |  |

The value of the Optional offer is not included in the anticipated value of public tender JN-B0410.

The prices of this optional offer  are fixed till the end of the guarantee period of the main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the purchase of the equipment, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of  training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

Payment conditions:

100 % within 60 days after the completed training and receipt of the relevant Invoice.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

## Proforma Invoice FOR LOT ……

FOR PUBLIC TENDER JN-B0410

The Bidder must enclose the detailed breakdown of prices for each item of the offer (for each bidden Lot separately, and also for main and for optional offers).

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

Bidder: ………………………………………………………………

Address: ......................................................................................

## Bidder’s Statement regarding penal acts accord. to Art. 42 of ZJN-2B

– for the Company

FOR PUBLIC TENDER JN-B0410

We herewith declare, under penal and material responsibility, that against our Company there have not been issued  legally binding judgments for penal acts that are defined in Penal Code (Official Gazette of the Republic of Slovenia, nos. 63/94, 70/94, 23/99, 60/99, 40/04, 37/05, and 17/06):

- crime gathering;

- forbidden receiving of gifts, forbidden giving of gifts, giving bribes, receiving of gifts for unlawful interventions and giving  gifts for unlawful interventions,

- fraud,  business fraud, imposture at gaining loan or benefits and denial of financial obligations,

- money laundering.

Furthermore we declare, under penal and material responsibility, that against our Company there have not been issued legally binding judgments for fraud against financial interests of European Communities (according to the 1. Art. of Convention on the protection of the European Communities’ financial interests).

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

Bidder's legal representative:

Name and surname: .................................................

## Bidder’s Statement regarding penal acts accord. to Art. 42 of ZJN-2B

– for legal representative

FOR PUBLIC TENDER JN-B0410

I, the undersigned, declare under penal and material responsibility, that against me there have not been issued  legally binding judgments for penal acts that are defined in Penal Code (Official Gazette of the Republic of Slovenia, nos. 63/94, 70/94, 23/99, 60/99, 40/04, 37/05, and 17/06):

- crime gathering;

- taking bribes at elections, forbidden receiving of gifts, forbidden giving of gifts, taking bribes, giving bribes, receipt of gifts for unlawful interventions and giving gifts for unlawful interventions,

- fraud,  business fraud, imposture at gaining loan or benefits and denial of financial obligations,

- money laundering.

Furthermore I declare under penal and material responsibility, that against me there have not been issued  legally binding judgments for fraud against financial interests of European Communities (according to the 1. Art. of Convention on the protection of the European Communities’ financial interests).

Place and date: 			Stamp:			             Signature:

........................................					             ...........................................

Bidder: ………………………………………………………………

Address: ......................................................................................

## Bidder's Statements given under penal and material responsibility

FOR PUBLIC TENDER JN-B0410

- We herewith declare, under penal and material responsibility, that we are registered by the competent entity of the state and have valid extract from judicial or other records, as well as all the necessary permits from the body competent for the activity which is the subject of this public tender.

- We herewith declare, under penal and material responsibility, that no compulsory settlement, bankruptcy or liquidation procedures have been initiated against our company. We have not suspended our business activities pursuant  to a court order or other compulsory order.

- We herewith declare, under penal and material responsibility, that our firm is reliable, capable of managing a business, has sufficient technical capabilities, experiences and reputation and has employees, who are qualified to carry out the Contract.

- We herewith declare, under penal and material responsibility, that we have fulfilled our obligations relating to the payment of taxes, social security contributions and other contributions or business obligations.

- We herewith declare, under penal and material responsibility, that we have fulfilled liabilities towards our subcontractors resulting from the previous public procurement procedures.

- We herewith declare, under penal and material responsibility, that during a period of five years prior to the publication of this public tender, there have not been issued  legally binding judgments for infringements connected with our business activities or issued with a court or administrative orders, prohibiting us from carrying out the activity, which is the subject of this public tender. Furthermore, we have not been sentenced and are not in penal proceedings on suspicion of bribery.

- We herewith declare, under penal and material responsibility, that we have paid taxes, contributions and other obligatory dues or business obligations in accordance with the regulations of the State where we have the headquarters. We have also paid all overdue business obligations that we have had in the Republic of Slovenia.

- We herewith declare, under penal and material responsibility, we have settled all our liabilities towards the RTV Slovenija and have brought no legal actions against the RTV Slovenija.

- We herewith declare, under penal and material responsibility, there are no legal limitations, which should preclude us from signing the Contract.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

Bidder: ………………………………………………………………

Address: ......................................................................................

## Declaration of the Bidder regarding the spare parts

FOR PUBLIC TENDER JN-B0410

We herewith declare under penal and material responsibility, that we are going to assure the necessary spare parts at least five (5) years from the date of final acceptance of the equipment which we offered on the public tender JN-B0410, according to the official spare parts price list.

We shall inform the Contracting Authority about every price change of  official spare parts price list.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

Bidder: ………………………………………………………………

Address: ......................................................................................

## Declaration of the Bidder – all the offered equipment is factory new

FOR PUBLIC TENDER JN-B0410

We herewith declare, under penal and material responsibility, that all the equipment, offered on the Public Tender JN-B0410,  is factory-new.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

Bidder: ………………………………………………………………..

Address: ......................................................................................

## Declaration of the BIdder regarding the software upgrades

FOR PUBLIC TENDER JN-B0410

We herewith declare, under penal and material responsibility, that all the software upgrades within the warranty period shall be free of charge for the Contracting Authority.

Place and date: 			Stamp:			             Bidder's official

representative or authorized person

........................................					.........................................................

Manufacturer of the equipment: ..................................................

Address: .......................................................

....................................................................

## STATEMENT OF THE MANUFACTURER

FOR PUBLIC TENDER JN-B0410

We,  ………(the Manufacturer of the equipment )  confirm, that we are acquainted with the contents of Public Tender JN-B0410 (Purchase of of studio cameras HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods), to which ……  (the Bidder) is going to submit his bid, with the Pro forma Invoice no.: ………

WE DECLARE, that ………… (the Bidder) has full access to the spare parts and replacement equipment for the bidden equipment, according to the requirements from the tender documentation no. JN-B0410.

Place and date: 			Stamp:			             Manufacturer's official

representative or authorized person

........................................					.........................................................

Principal: ..................................................

Address: .......................................................

....................................................................

## STATEMENT OF THE PRINCIPAL

FOR PUBLIC TENDER JN-B0410

We,  ………(the Principal of the equipment )  confirm, that we are acquainted with the contents of Public Tender JN-B0410 (Purchase of of studio cameras HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods), to which ……  (the Bidder) is going to submit his bid, with the Pro forma Invoice no.: ………

WE DECLARE, that ………… (the Bidder) has full access to the spare parts and replacement equipment for the bidden equipment, through our firm, according to the requirements from the tender documentation no. JN-B0410.

As evidence of the aforesaid we enclose the copy of the Contract between us and the Manufacturer of the equipment, wherein it is evident that we have full access to the spare parts and replacement equipment for the bidden equipment.

In case the Contracting Authority subsequently requests the original Contract, we shall present the same for verification.

Place and date: 			Stamp:			             Principal's official

representative or authorized person

........................................					.........................................................

## Sample Contract

Each article in the Contract is written first in Slovenian and then in English language. In case of any doubt or misunderstanding, the Slovenian version should therefore be considered final.

POGODBA ŠTEV.   /  CONTRACT  NO.

JN-B0410

Nakup SD in HD studijskih kamer, HD kamkorderjev, objektivov za studijske kamere in kamkorderje, studijskih stativov in ENG stativov

Purchase of SD and HD studio cameras HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods

Sklenjena med:

Concluded between:

Radiotelevizija Slovenija, Javni zavod

Kolodvorska 2

1550 Ljubljana

Slovenia

Identifikacijska štev. za DDV / ID V.A.T. No.: SI29865174

Matična štev. / Corporate registry No.:5056497

ki ga zastopa generalni direktor mag. Marko Filli

represented by mag. Marko Filli

(v nadaljevanju teksta “NAROČNIK”)

(hereinafter referred to as “CONTRACTING AUTHORITY”)

in / and

……………………………………

……………………………………

……………………………………

……………………………………

Identifikacijska štev. za DDV / ID V.A.T. No.: ……………………………………

Matična štev. / Corporate registry No.: ……………………..

ki ga zastopa …………………………………………………..

represented by …………………………………………………

(v nadaljevanju teksta “PONUDNIK”)

(hereinafter referred to as “BIDDER”)

1. člen

Pogodbeni stranki ugotavljata, da je naročnik izvedel postopek oddaje javnega naročila na osnovi 25. člena Zakona o javnem naročanju – ZJN-2B (Ur. list RS, štev. 19/2010) za nakup: SD in HD studijskih kamer, HD kamkorderjev, objektivov za studijske kamere in kamkorderje, studijskih stativov in ENG stativov – po sklopih.

Ponudnik je bil izbran kot najugodnejši ponudnik za sklop………, na osnovi javnega naročila, objavljenega na portalu javni naročil RS, štev. ….., z dne …………  in v Uradnem listu EU, štev. …., z dne ………...

Article 1

The Parties agree that the Contracting Authority carried out the procedure related to the submission of Public Procurement based on Article 25. of the Public Procurement Act (Official Gazette of the Republic of Slovenia, No. 19/2010) for the purchase of: SD and HD studio cameras HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods - according to the Lots.

The Bidder has been selected as the most favourable Bidder for Lot….., based on the Public Tender, published in the in the TED of Republic of Slovenia, on … under the no. … and  in the Official Gazette of EU, No. …. on  …

2. člen

S to pogodbo se pogodbeni stranki dogovorita o splošnih in posebnih pogojih izvedbe javnega naročila. Sestavni del te pogodbe so pogoji, določeni z razpisno dokumentacijo štev. JN-B0410, sprejeti in navedeni v ponudbeni dokumentaciji  .............. z dne ……..

Article 2

The Parties agree on general and special conditions related to the realization of the Public Tender. All conditions determined by the Tender documentation no. JN-B0410, adopted and specified in the Bid documentation ….dated   …… shall constitute an integral part of this Contract.

3. člen

3.1

Končna vrednost nakupa ………………………………..,  znaša:

EUR …………………………. DDP Ljubljana (skladno z INCOTERMS 2000), dostavljeno v skladišče

RTV Slovenija, Čufarjeva 6, Ljubljana;  brez DDV

V primeru plačila domačemu ponudniku, bo le-ta ob izstavitvi računa priračunal 20 % DDV.

V primeru pridobitve blaga znotraj Skupnosti (EU), bo naročnik izvršil samoobdavčitev z DDV, skladno z veljavno zakonodajo o DDV.

Cene po tej pogodbi so fiksne in se ne smejo spreminjati do konca trajanja te pogodbe.

3.2

Vsi dokumenti, ki jih izda ponudnik, morajo vsebovati tudi oznako javnega naročila oz. pogodbe in oznako naročila. Naročilo bo ponudnik prejel po pričetku veljavnosti pogodbe.

Naročnik bo navedeni znesek plačal na  račun ponudnika štev.:.......................................pri banki: ...........................................................;  v skladu z naslednjimi plačilnimi pogoji:

- 80 %  -  v  roku 60 dni po naročnikovem količinskem prevzemu dobavljene opreme in prejemu ustreznega računa;

- 20 % - v 60 dneh po uspešno opravljenem končnem prevzemu  dobavljene opreme in predložitvi zapisnika, podpisanega s strani naročnika;

Naročnik ne bo predložil garancije za zavarovanje odloženih plačil, kakor tudi ne bo predložil kakšnega drugega instrumenta za zavarovanje plačil.

Article 3

3.1

The total value of the purchase of …………………………………………………………..  amounts to:

EUR …………………………. DDP Ljubljana (accord. to INCOTERMS 2000), delivered to the

warehouse of RTV Slovenija, 6, Čufarjeva Street, Ljubljana; without VAT

In case of a payment to a domestic Bidder, the 20 % VAT shall be added.

In case of a purchase of the goods on the internal EU market, the Contracting Authority  shall carry out the reverse charge of VAT,  according to the valid  Law on VAT

The prices of the equipment according to this Contract are fixed and can not be changed for the period of the Contract duration.

3.2

All documents issued by the Bidder (Invoices, Delivery Notes) shall also incorporate the Public Tender or Contract Number and the Order number. The Bidder will receive the order after the beginning of the contract validity.

The Contracting Authority shall pay the amount specified in favour of the Bidder’s account No …………………………………… at the Bank ………………………………………………….in accordance with the following payment terms:

- 80 % - within 60 days after the quantitative acceptance of delivered equipment has been carried out by Contracting Authority  and the relevant invoice has been received;

- 20 %  -  within 60 days after the final acceptance of delivered equipment has been successfully carried out and the protocol  has been signed by the Contracting Authority;

The Contracting Authority shall not present bank guarantee or any other instrument for insurance of the retained payments.

4. člen

Ponudnik bo izpolnil naročilo v skladu s pogoji ponudbene dokumentacije, ki je sestavni del te pogodbe, v količini, kvaliteti in rokih, sicer bo naročnik unovčil garancijo za dobro izvedbo pogodbenih obveznosti.

Article 4

The Bidder shall fulfil the order according to the terms of the Bid documentation, being the constituent parts of this Contract, in terms of quantities and deadlines required, otherwise the Contracting Authority shall cash-in a performance guarantee.

5. člen

Ponudnik bo blago (opremo) dobavil najkasneje v roku …………………. tednov od dneva pričetka veljavnosti te pogodbe. Po tem roku naročnik lahko unovči garancijo za dobro izvedbo pogodbenih obveznosti.

Kosovni prevzem opreme bo opravil naročnik v skladišču RTV Slovenija v Ljubljani. S  kosovnim prevzemom se ugotavlja celovitost dobavljene opreme.

Kosovni prevzem opreme mora biti opravljen najkasneje v štirih delovnih dneh po dobavi v skladišče RTV Slovenija.

Article 5

Bidder shall supply the goods (equipment) at the latest within .….........weeks after the Contract validity date. After this delivery deadline the Contracting Authority may cash-in a performance guarantee.

The quantitative acceptance of the equipment will be done by Contracting Authority in the warehouse of RTV Slovenija in Ljubljana. With the quantitative acceptance the completeness of delivered equipment will be determined.

The quantitative acceptance must be done within four working days after the equipment is delivered to the warehouse of RTV Slovenija, at the latest.

6. člen

6.1

Garancijski pogoji so navedeni v ponudbeni dokumentaciji, ki je sestavni del te pogodbe in so bili sprejeti s strani obeh pogodbenih partnerjev. Garancijski rok traja ……. mesecev in prične teči z dnem, ko naročnik podpiše zapisnik o uspešno opravljenem končnem prevzemu opreme (blaga).

Ponudnik se obvezuje tudi, da bo naročniku v času garancije nudil vso podporo v primeru, ko le-ta sam ne bo mogel  odpraviti napake na opremi. Naslov evropskega servisa za ponujeno opremo je: Kontaktna oseba ponudnika je..................., telefon:................fax:..........e-mail:...................... Ponudnik je dolžan naročniku takoj sporočiti morebitne spremembe glede kontaktne osebe.

6.2

Ponudnik zagotavlja za čas garancijskega roka in 5 let po izvedenem končnem prevzemu:  odpošiljanje nadomestnega modula max. v 2  tednih (48 ur za dele, ki so v Evropskem skladišču) po zahtevi, upoštevajoč delovne dni. Zahteva naročnika bo poslana preko elektronske pošte ali fax sporočila.  Naročnik bo vrnil defektni modul po prejetju nadomestnega.

6.3

Neupravičene zamude pri odpremi rezervnih delov ali nadomestne opreme bodo upoštevane kot negativna referenca v bodočih razpisih, kjer bo sodeloval izbrani ponudnik. Naročnik bo štel kot negativno referenco več kot trikratno zamudo pri dobavi rezervnih delov ali nadomestne opreme glede na roke iz tč. 6.2.

Ponudnik z negativno referenco ne bo smel sodelovati v bodočih razpisih, ki jih bo objavil naročnik, in sicer za dobo treh let od ugotovljene negativne reference.

6.4

Ponudnik zagotavlja, da bodo vse nadgradnje programske opreme v času garancijskega roka brezplačne za naročnika.

Article 6

6.1.

The guarantee terms are determined in the Bid documentation, which shall constitute the integral part of this Contract and have been adopted by both Parties. The guarantee period is….   months and shall start with the date the Contracting Authority has signed the Protocol of the final acceptance of the equipment (goods).

The Bidder is obliged to give the Contracting Authority all the support within the guarantee period, in case he would not be able to eliminate the errors on the equipment. The address of the European Service is: …………………….. The contact person: ..............., telephone: ................, fax: ...................., e-mail: .............

All changes regarding the contact person shall be promptly announced to the Contracting Authority.

6.2

The Bidder ensures for the warranty period and until 5 years after final acceptance:  the dispatch of the replacement module by express courier service within max.  2 weeks (48 hours for spare parts which are on stock in Europe) after notice in working days. The Contracting  Authority will report the fault by fax or e-mail. The Contracting Authority will return the defective module after receipt of the replacement one.

6.3

Unjustified delays at error elimination with regard to art. 6.2 of this Contract are going to be considered as negative reference in the future Public Tenders, where the selected Bidder from this Public Tender will participate. The Contracting Authority is going to consider as negative reference if unjustified delays, mentioned in the previous sentence happen more than three times.

The Bidder with negative reference will not be permitted to participate in the future Public Tenders, published by the Contracting Authority, for the period of three years from the date the negative reference has been ascertained.

6.4

Within the warranty period the Bidder assures free of charge delivery of all new versions of software for the equipment that is the subject of this Contract.

7. člen

Končni prevzem bo uspešno opravljen, ko bo naročnik ugotovil, da vsa oprema deluje v skladu z v razpisnih pogojih zahtevanimi tehnološkimi in funkcionalnimi lastnostmi. Ob končnem prevzemu opreme je lahko prisoten predstavnik ponudnika.

Naročnik mora opraviti končni prevzem dobavljenega blaga (opreme) najkasneje v roku 21 (enaindvajset) dni od dneva kosovnega prevzema blaga.

Vse morebitne nepravilnosti, ugotovljene pri končnem prevzemu opreme, morajo biti odpravljene s strani ponudnika v roku 14 (štirinajst) dni. Po tem roku naročnik lahko unovči garancijo za dobro izvedbo pogodbenih obveznosti.  Šteje se, da je končni prevzem izvršen takrat, ko so odpravljene vse morebitno ugotovljene nepravilnosti, ki jih mora naročnik specificirati v pisni obliki.

Article 7

The final acceptance of the equipment will be successfully done, when the Contracting Authority ascertains that all the equipment has been installed and is working according to the requested technical and functional requirements. At the final acceptance procedure the participant of the Bidder can be present.

The final acceptance of the goods (equipment) must be completed within 21 (twenty-one) days after the quantitative acceptance date, at the latest.

All eventual incorrectness, ascertained at final acceptance procedure, are to be eliminated by the Bidder within 14 (fourteen) days, after this time limit the Contracting Authority may cash-in a performance guarantee. It is understood that the final acceptance will be completed when all eventual incorrectness are eliminated. The incorrectness are to be specified in writing by the Contracting Authority.

8. člen

Ponudnik se obvezuje, da bo v dobi zagotavljanja rezervnih delov (pet let po končnem prevzemu opreme), dobavljal le-te skladno z uradnim cenikom ponudnika. Ponudnik mora ob vsaki spremembi uradnega cenika rezervnih delov,  o tem obvestiti naročnika.

Article 8

The Bidder is obliged that he shall in the period of five years after the date of final acceptance of the equipment, deliver  the necessary spare parts in accordance with the official price-list. The Bidder shall inform the Contracting Authority about every price change of  official spare parts price list.

9. člen

Naročnik lahko oporeka kakovosti blaga v zakonsko določenih rokih po tem, ko ugotovi neustrezno kakovost. V garancijskem roku pa lahko vsak čas zahteva odpravo napake, ki jo je ponudnik dolžan odpraviti. Pogoji odpravljanja napak so navedeni v ponudbeni dokumentaciji, ki je sestavni del te pogodbe. V primeru, da kakovost blaga ni v skladu z zahtevami razpisne dokumentacije ali iz ponudbene dokumentacije, bo naročnik unovčil garancijo za dobro izvedbo pogodbenih obveznosti.

Ponudnik  odgovarja za  skrite napake na dobavljenem blagu. Skrite napake so tiste , ki  jih  naročnik ni mogel odkriti  z  običajnim pregledom  dobavljenega blaga. Naročnik mora obvestiti  ponudnika o napaki v osmih dneh, šteto od dneva, ko je napako opazil.

Article 9

After the Contracting Authority has established the inappropriate quality of goods, this quality of goods may be contested in statutory deadlines. During the guarantee period the elimination of defects can be requested at any time and the Bidder shall be liable to remedy any defect. Terms related to the remedy of defects are determined in the Bid documentation, which constitute an integral part of this Contract. If the quality of goods is not in accordance with the stipulations of the tender or bid documentation, the Contracting Authority shall cash-in a performance guarantee.

The Bidder is responsible for the hidden errors on the delivered goods. Hidden errors are those, the Contracting Authority was unable to determine with the commonly performed examination of the delivered goods. The Contracting Authority shall inform the Bidder about the hidden error within eight days, counting from the date the hidden error had been noticed.

10. člen

Če dobava ni realizirana v predvidenem roku po ponudnikovi krivdi, razen v primeru višje sile, je ponudnik dolžan plačati naročniku kazen v višini 1 odstotek vrednosti dobave, ki predstavlja funkcionalno enoto, za vsak zaključen teden, vendar ne več kot 5 odstotkov celotne pogodbene vrednosti.  Kazen začne teči  naslednji delovni dan po določenem času dobave.

Če dobava zaostaja po ponudnikovi krivdi za več kot 6 tednov, ima naročnik pravico, razen če uveljavlja kazenski zahtevek, da določi nov dobavni rok ali prekine pogodbo.

Article 10

If the goods are not supplied due to the Bidder’s fault in a planned timescale, with the exception of Force Majeure, the Bidder shall be liable to pay penalties in the amount of 1 per cent of a respective partial delivery, incorporating a functional unit, for each week completed, but not increasing 5 % of the total Contract amount. The penalty shall commence next working day after the delivery deadline determined.

If the delivery is delayed due to Bidder’s fault for more than 6 weeks, the Contracting Authority shall have right, unless the penal claim is exercised, to determine a new delivery time or to terminate the Contact.

11. člen

11.1

Ponudnik je skladno z določili razpisne dokumentacije štev. JN-B0410 ponudil ločeno od osnovne ponudbe za sklop A tudi ceno vzdrževalnega tečaja za kamere (opcija 1). Cene opcijske ponudbe so fiksne do poteka garancijske dobe za opremo osnovne ponudbe sklopa A.

V primeru, da se bo naročnik odločil za vzdrževalni tečaj že ob odločitvi o oddaji javnega naročila, ga bosta  pogodbeni stranki vključili v osnovno pogodbo. Če pa se bo naročnik odločil za vzdrževalni tečaj kasneje (v roku do poteka garancijske dobe), bosta pogodbeni stranki sklenili Dodatek k pogodbi JN-B0410, za katerega bo osnova ta opcijska ponudba.

Dodatek mora biti podpisan in žigosan s strani obeh pooblaščenih pogodbenih predstavnikov.

11.2

Ponudnik je skladno z določili razpisne dokumentacije štev. JN-B0410 ponudil ločeno od osnovne ponudbe za sklop B tudi ceno vzdrževalnega tečaja za kamkorderje tipa 1 in 2 (opcija 2), ceno vzdrževalnega tečaja za rekorderje (opcija 3) in ceno vzdrževalnega tečaja za kamkorderje tipa 3 in 4 (opcija 4). Cene opcijskih ponudb so fiksne do poteka garancijske dobe za opremo osnovne ponudbe sklopa B.

V primeru, da se bo naročnik odločil za enega ali več vzdrževalnih tečajev  že ob odločitvi o oddaji javnega naročila, ga/jih bosta  pogodbeni stranki vključili v osnovno pogodbo. Če pa se bo naročnik odločil za enega ali več vzdrževalnih tečajev  kasneje (v roku do poteka garancijske dobe), bosta pogodbeni stranki sklenili Dodatek k pogodbi JN-B0410, za katerega bodo osnove opcijske ponudbe 2, 3 in 4.

Dodatek mora biti podpisan in žigosan s strani obeh pooblaščenih pogodbenih predstavnikov.

Nadalje je ponudnik skladno z določili razpisne dokumentacije štev. JN-B0410 ponudil ločeno od osnovne ponudbe za sklop B tudi opcijsko ponudbo opreme – dodatna kamkorderja tip 5 (opcija 5). Cene opcijske ponudbe so fiksne do poteka garancijske dobe za opremo osnovne ponudbe sklopa B.

V primeru, da se bo naročnik odločil za nakup dodatne opreme (opcija 5), bosta pogodbeni stranki sklenili Dodatek k pogodbi JN-B0410, za katerega bo osnova opcijska ponudba 5, ki je priložena k tej pogodbi. Dodatek mora biti podpisan in žigosan s strani obeh pooblaščenih pogodbenih predstavnikov.

11.3

Ponudnik je skladno z določili razpisne dokumentacije štev. JN-B0410 ponudil ločeno od osnovne ponudbe za sklop D tudi ceno vzdrževalnega tečaja za objektive (opcija 6). Cene opcijske ponudbe so fiksne do poteka garancijske dobe za opremo osnovne ponudbe sklopa D.

V primeru, da se bo naročnik odločil za vzdrževalni tečaj že ob odločitvi o oddaji javnega naročila, ga bosta  pogodbeni stranki vključili v osnovno pogodbo. Če pa se bo naročnik odločil za vzdrževalni tečaj kasneje (v roku do poteka garancijske dobe), bosta pogodbeni stranki sklenili Dodatek k pogodbi JN-B0410, za katerega bo osnova ta opcijska ponudba.

Dodatek mora biti podpisan in žigosan s strani obeh pooblaščenih pogodbenih predstavnikov.

11.4

Ponudnik je skladno z določili razpisne dokumentacije štev. JN-B0410 ponudil ločeno od osnovne ponudbe za sklop G tudi ceno vzdrževalnega tečaja za objektive (opcija 7). Cene opcijske ponudbe so fiksne do poteka garancijske dobe za opremo osnovne ponudbe sklopa G.

V primeru, da se bo naročnik odločil za vzdrževalni tečaj že ob odločitvi o oddaji javnega naročila, ga bosta  pogodbeni stranki vključili v osnovno pogodbo. Če pa se bo naročnik odločil za vzdrževalni tečaj kasneje (v roku do poteka garancijske dobe), bosta pogodbeni stranki sklenili Dodatek k pogodbi JN-B0410, za katerega bo osnova ta opcijska ponudba.

Dodatek mora biti podpisan in žigosan s strani obeh pooblaščenih pogodbenih predstavnikov.

Article 11

11.1

The Bidder offered according to the stipulations of the tender documentation no. JN-B0410, separately from the main offer  for Lot A also the prices for training course for cameras (option 1). The prices of this optional offer  are fixed till the end of the guarantee period of the Lot A main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the Decision on the award of a Contract, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of  training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

The Annex must be signed and duly stamped by the authorized representatives of both Parties.

11.2

The Bidder offered according to the stipulations of the tender documentation no.  JN-B0410, separately from the main offer  for Lot B also the prices for training course for camcorders type 1 and type 2 (option 2),  training course for recorders (option 3) and the price  for training course for camcorders type 3 and type 4 (option 4),  The prices of these optional offers are fixed till the end of the guarantee period of the Lot B main offer equipment.

In case the Contracting Authority decides for the purchase of one or more training courses already at the Decision on the award of a Contract, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of  one or more training courses later (in the period till the end of the guarantee period of the equipment), the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be the optional offers 2. 3 and 4.

The Annex must be signed and duly stamped by the authorized representatives of both Parties.

Furthermore the Bidder offered according to the stipulations of the tender documentation no. JN-B0410, separately from the main offer for Lot B also the prices for additional equipment – camcorders type 5 (option 5). The prices of the optional offer are fixed till the end of the guarantee period of the Lot B main offer equipment.

In case the Contracting Authority decides for the purchase of additional equipment  (option 5), the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be the optional offer, that is enclosed to this Contract.

The Annex must be signed and duly stamped by the authorized representatives of both Parties.

11.3

The Bidder offered according to the stipulations of the tender documentation no. JN-B0410, separately from the main offer  for Lot  D also the prices for training course for lenses (option 6). The prices of this optional offer  are fixed till the end of the guarantee period of the Lot D main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the Decision on the award of a Contract, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of  training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

The Annex must be signed and duly stamped by the authorized representatives of both Parties.

11.4

The Bidder offered according to the stipulations of the tender documentation no. JN-B0410, separately from the main offer  for Lot  G also the prices for training course for lenses (option 7). The prices of this optional offer  are fixed till the end of the guarantee period of the Lot G main offer equipment.

In case the Contracting Authority decides for the purchase of  training course already at the Decision on the award of a Contract, the same will be included in the basic Contract. In case the Contracting Authority decides for the purchase of  training course later (in the period till the end of the guarantee period of the equipment)  the contractual parties shall conclude an Annex to the Contract  JN-B0410 in writing. The basis for this Annex shall be this optional offer.

The Annex must be signed and duly stamped by the authorized representatives of both Parties.

12. člen

Pogodbeni stranki se obvezujeta, da bosta naredili vse, kar je potrebno za izvršitev pogodbe in da bosta ravnali kot dobra gospodarja. Vse  morebitne spore v zvezi s to pogodbo bosta stranki najprej poizkušali rešiti po mirni poti. Če razrešitev po mirni poti ne bo mogoča, sta pogodbeni stranki  sporazumni, da je za reševanje vseh morebitnih sporov iz te pogodbe pristojno sodišče v Ljubljani po slovenskem pravu.

Article 12

The Parties undertake to exercise their best efforts to implement the Contract and act with due diligence. The Parties shall endeavour to resolve all eventual disputes arising out of this Contract by mutual agreement. Should such amicable settlement not be substantially reached, the Parties agree to refer, the dispute arising out of this Contract, to the Court of Ljubljana, according to the Slovenian law.

13. člen

V primeru višje sile kot so požar, poplava, potres, vojna, stavke in podobno, se lahko naročnik in ponudnik sporazumeta za podaljšanje roka izpolnitve po tej pogodbi, za dobo trajanja višje sile. Če zaradi daljših rokov trajanja višje sile ne bi bilo mogoče izpolniti pogodbenih obveznosti, lahko pogodbeni stranki pogodbo prekineta. Prekinitev ali podaljšanje  pogodbe mora biti v pisni obliki. V primeru daljšega trajanja višje sile pogodbeni stranki lahko pogodbo razdreta, vsaka stranka nosi svoje, do razdrtja pogodbe nastale stroške.

Article 13

In the event of Force Majeure such as fire, flood, earthquake, war, strikes and similar, the Contracting Authority and the Bidder agree to extend the period that its performance of such obligations is prevented by the event of Force Majeure. Should it not be possible to fulfil obligations under this Contract in spite of all longer periods of time, the Parties may terminate the Contract.The termination or extension of the Contract shall be provided in writing. In the case of longer period of Force Majeure the Parties may terminate the Contract, each party bearing their own costs, that have been occurred till then.

14. člen

Pogodba je sklenjena z odložnim pogojem in stopi v veljavo:

- z dnem obojestranskega podpisa  s strani pooblaščenih podpisnikov naročnika in ponudnika ter veljavnim žigom obeh pogodbenih strank;

- s predložitvijo garancije za dobro izvedbo pogodbenih obveznosti (izdane s strani banke ali zavarovalnice), brezpogojne, nepreklicne in plačljive na prvi poziv, v višini 10 % pogodbene vrednosti in z veljavnostjo še šest mesecev po pogodbeno določenem datumu dobave opreme. Ponudnik je dolžan predložiti garancijo za dobro izvedbo pogodbenih obveznosti najkasneje 10 dni po podpisu pogodbe.

Article 14

The agreement is made under the suspense and enters into force:

- on the day the authorized representatives of the Contracting Authority and Bidder affix their signatures and valid stamps of both parties;

- by Bidder’s presentation of the  Performance Guarantee (issued by Bank or Insurance Company), irrevocable, unconditional and payable at first call in the amount of 10 % of the Contractual value and valid six months after confirmed delivery of the equipment. The Bidder must present the Performance Guarantee at the latest 10 days after the signature of the Contract.

15. člen

Določila pogodbe so fiksna in se jih ne sme spreminjati v času trajanja pogodbe. V primeru, da nastanejo izredni pogoji, ki bi onemogočali izpolnjevanje pogodbenih določil, lahko stranki skleneta dodatek k pogodbi v pisni obliki. Ta mora biti podpisan in žigosan s strani obeh pooblaščenih pogodbenih predstavnikov.

Article 15

The provisions of the Contract are fix and can not be amended during the duration of the Contract. In the event of extraordinary conditions that should preclude the fulfilment of the Contract provisions, the Parties may conclude the Annex to the Contract in writing. This Annex must be signed and duly stamped by the authorized representatives of the Parties.

16. člen

Pogodbeni stranki izjavljata , da je to pogodbo možno odpovedati pred  njeno izpolnitvijo  v primeru, da nastopi  ena od  naslednjih spodaj navedenih  okoliščin.

Naročnik  lahko odpove pogodbo pred njeno izpolnitvijo :

- če je proti  ponudniku uveden stečaj ali prisilna poravnava,

- če je neuspešno pretekel  pogodbeno določen rok, pa ponudnik ni izpolnil pogodbenih obveznosti,

- če ponudnik  ni odpravil ugovarjanih napak  oz. odprava napak ni možna,

Ponudnik lahko odpove pogodbo :

- če naročnik ne plača  pogodbene cene.

Odpoved pogodbe se poda v pisni obliki, pogodba preneha veljati  v petnajstih  dneh   po prejemu takšne odpovedi, podpisane s strani pooblaščene osebe pogodbenega stranke.

Če odpove pogodbo naročnik, lahko zahteva povrnitev škode, ki mu je nastala zaradi neizpolnitve pogodbe s strani ponudnika. Če odpove pogodbo  ponudnik, lahko zahteva plačilo za že opravljene storitve do dneva odpovedi pogodbe. Odškodninski zahtevki pogodbeno zveste stranke s tem niso izključeni.

Article 16

The Contract parties declare this contract could be terminated before its fulfilment only if one of the following circumstances occurred.

The Contracting Authority is justified to terminate the contract before its fulfilment if:

- the compulsory settlement, bankruptcy or liquidation procedures are initiated against the  Bidder,

- the contractual delivery date is expired  and the Bidder 's obligations are not fulfilled,

- the Bidder does not eliminate the defects or the elimination of the defects is not possible.

The Bidder is justified to terminate the contract if:

- the Contracting Authority does not pay the contractual price.

The termination of the Contract shall be provided in writing, the contract ceases to be valid fifteen days from the receipt of such termination, signed by the authorized person of either party. If the Contract is terminated by the Contracting Authority, the Bidder is liable to reimburse the damage, caused due to his non-fulfilment of the Contract.  If the Contract is terminated by the Bidder, he is justified to require the payment for contractual works already done till the date of the contract termination. By this the indemnity claims of contractual loyal party are not  excluded.

17. člen

S strani naročnika je za izvajanje pogodbe odgovoren …. (telefon: 01 475 ….), s  strani ponudnika pa: …. (telefon: ….).

Article 17

Each party shall nominate representatives, responsible for the  realization of the Contract, i.e.: for the Contracting Authority (phone: +386 1 475  ; for the Bidder: _______________ (phone: +___________)

18. člen

Pogodba je sestavljena iz naslednjih delov, ki predstavljajo enovito celoto:

- Osnovna pogodba s komercialnimi pogoji,

- Priloga 1 – Ponudbena dokumentacija štev. ............................................................

- Priloga 2 – Garancija za dobro izvedbo pogodbenih obveznosti

Vsak člen pogodbe je najprej napisan v slovenskem jeziku, nato pa še v angleškem jeziku. V primeru kakršnegakoli dvoma ali napačne razlage, velja slovenska verzija.

Pogodba je sestavljena v petih izvodih, od katerih prejme naročnik tri izvode, ponudnik pa dva izvoda.

Article 18

The Contract encompass the following parts, which represent the uniform whole:

- Basic Contract including commercial terms, Slovenian and English version

- Enclosure 1 - Bid documentation …..No.   …………

- Enclosure 2 - Performance Guarantee

Each article in the Contract is written first in Slovenian and then in English language. In case of any doubt or misunderstanding, the Slovenian version should therefore be considered final.

The Contract is drawn up in five copies, the Contracting Authority receiving three copies, and the Bidder two copies.

PODPIS IN ŽIG POOBLAŠČENIH PREDSTAVNIKOV NAROČNIKA IN PONUDNIKA – POGODBA ŠT. JN-B0410 THE SIGNATURE AND SEAL OF THE AUTHORIZED REPRESENTATIVES OF THE CONTRACTING AUTHORITY AND BIDDER – CONTRACT No. JN-B0410

Ljubljana, …....................	………….., …….…................

NAROČNIK: /	PONUDNIK: /

CONTRACTING AUTHORITY:	BIDDER:

Radiotelevizija Slovenija, Javni zavod	…………………………………..

Generalni direktor 	…………………………………..

Director General	…………………………………..

mag. Marko Filli	…………………………………..

Žig / Stamp:	Žig / Stamp:

Place and date....................................

Beneficiary……………………..

## Guarantee for the seriousness of Bid No......

In accordance with the public procurement, published in  the Official Gazette of the Republic of Slovenia, no. …..,  dated ………, for purchase of the following goods (services):

1…………………………..

2…………………………..

3…………………………..

for the purposes of the Contracting Authority (i.e. Beneficiary), the Bidder ………………………………. is bound to submit to the Contracting Authority a guarantee for seriousness of the bid to the amount of ………….  EUR.

The Bank is bound to pay stated amount in the following events:

a)  the Bidder withdraws or changes his bid within its term of validity

or

b)  the Bidder, selected at public selection of bids:

- does not fulfil or withdraws from signing a contract in accordance with tender documentation

or

- does not submit or refuses to submit a Performance Bank Guarantee in accordance with tender documentation.

We are bound to pay within 15 days after the receipt of the first written demand by Contracting Authority the above mentioned amount without any additional grounding if the Contracting Authority in it’s claim states, that it is entitled to demanded amount due to fulfilment of one or two above mentioned events and if the Contradicting Authority states, to which event it refers.

Claim for above reason must be presented to the Bank and must contain:

1. original summons of Contracting Authority for cashing-in the Bank guarantee in accordance with

above paragraph and

2. original Guarantee no. ../……………..

With every payment on this guarantee the sum of guarantee is reduced by the same amount.

This guarantee is valid until the Bidder is chosen according to the above mentioned public procurement and (in case the correct or partly correct offer is accepted) until the Contract between chosen Bidder and Contracting Authority is signed and Performance Guarantee being presented by the chosen Bidder, but not later than the Bid expiration date .

We have to receive your claims for cashing-in of the amount guaranteed until 31 December, 2010 at the latest. After this term the validity of guarantee is automatically terminated irrespective of whether or not the guarantee is returned to the bank.

This guarantee is not transferable.

Eventual dispute between Beneficiary and the Bank will be settled by the Court of Ljubljana, according to the Slovenian law.

Bank

(stamp and signature)

Name of the Bidder: …………………………………………………………………

Address of the Bidder: ………………………………………………………………

## STATEMENT regarding the Performance Guarantee

FOR PUBLIC TENDER JN-B0410

Should we, .................. (The Bidder) be awarded  the Tender of RTV Slovenija, No. JN-B0410 – purchase of  SD and HD studio cameras HD camcorders, lenses for studio cameras and camcorders, studio and ENG tripods

WE DECLARE,

we will supply a Performance Bank Guarantee for 10 % of the contractual value upon Contract signature. The Performance Guarantee will be issued by the first class Bank or Insurance Company and will be irrevocable, unconditional and payable at first call,  valid 6 more months after the confirmed delivery date.

Place and date: 	Stamp:	Signature of the Bidder’s

authorized person

………………………………		………………………………

Radiotelevizija Slovenija, Javni zavod,

Commercial Department

e-mail: javna.narocila@rtvslo.si

Fax No..: + 386 1  475 2186

Date: .............................................................

## Explanation of the Tender documentation No. JN-B0410

1. Questions:

..................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................

2. Answers:

...................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................................